Table 1 - Operating parameters for the voltammetric analysis (Differential Pulse Voltammetry and Adsorptive Stripping Voltammetry) of the solutions obtained from the ashes of potatoes samples.

| Analytes | Tecniques | Electrolytes | Reagent |
| --- | --- | --- | --- |
| Pt | DPV/a | H2SO4 1 M | [N2H4SO4] = 1.2 mmol L-1, [H2CO] = 0.6 mmol L-1 |
| Rh | DPSAV | HCl 0.42 M | [H2CO] = 0.02 mol L-1 |

Table 2 - Operating parameters for the Differential Pulse Voltammetry and Adsorptive Stripping Voltammetry analysis of the solutions obtained from the potatoes samples.

| Parameter | Pt | Rh |
| --- | --- | --- |
| Initial potential (mV) | -300 | -900 |
| Final potential (mV) | -1000 | -1200 |
| Current range | Automatic | Automatic |
| Potential scan rate (mV s-1) | 50 | 10 |
| Potential of deposition (mV) | - | -700 |
| Cycle n° | 1 | 1 |
| Deposition time (s) | - | 30 |
| Stirring rate (r.p.m.) | 300 | 300 |
| Size of the drop (a.u.) | 60 | 60 |
| Delay time before potential sweep (s) | 10 | 10 |
| Working electrode | Hanging mercury drop electrode |  |
| Auxiliary electrode | Glassy carbon |  |
| Reference electrode | Ag/AgCl/KCl (sat) |  |
| Flowing gas | Nitrogen (99.998%) |  |

Table 3 - Platinum and rhodium concentrations in potatoes samples

| Sample | Origin | Pt (μg/Kg) | R.S.D. % ± | Rh (μg/Kg) | R.S.D. % ± |
| --- | --- | --- | --- | --- | --- |
| 1 | Italy - Sicily | 16 | 2.2 | 0.015 | 5.4 |
| 2 | Italy - Sicily | 8.9 | 6.5 | 0.016 | 6.5 |
| 3 | Italy - Sicily | 56 | 2.4 | 0.0008 | 11 |
| 4 | Italy - Sicily | 6.7 | 6.1 | 0.0086 | 4.2 |
| 5 | Italy - Sicily | 65 | 1.0 | 0.0008 | 10 |
| 6 | Italy - Sicily | 109 | 0.5 | 0.0008 | 11 |
| 7 | Francia | 13 | 6.2 | 0.0008 | 20 |
| 8 | Italy - Veneto | 69 | 4.0 | 0.0008 | 12 |
| 9 | Italy - Lazio | 20 | 3.5 | 0.0008 | 9.9 |
| 10 | Italy - Marche | 12 | 5.2 | 0.0008 | 16 |
| 11 | Italy - Emilia | 12 | 4.7 | 0.0008 | 4.9 |
| 12 | Italy - Emilia | 17 | 2.3 | 0.0008 | 14 |
| 13 | Italy - Sicily | 23 | 2.4 | 0.0008 | 13 |
| 14 | Francia | 21 | 1.0 | 0.0008 | 13 |
| 15 | Italy- Umbria | 1.3 | 5.6 | 0.0008 | 5.6 |
| 16 | Belgio | 0.007 | 8.0 | 0.017 | 8.0 |
| 17 | Italy - Campania | 0.007 | 11 | 0.0008 | 21 |
| 18 | Italy - Puglia | 0.007 | 10 | 0.0008 | 25 |
| 19 | Australia | 0.007 | 12 | 0.0008 | 12 |
| 20 | Italy - Abruzzo | 0.007 | 16 | 0.0008 | 15 |
| 21 | Italy - Sicily | 0.007 | 12 | 0.0008 | 16 |
| 22 | Italy - Sicily | 0.21 | 3.2 | 0.0008 | 3.2 |
| 23 | Emilia Romagna | 0.007 | 12 | 0.0303 | 6.1 |
| 24 | Italy - Sicily | 0.007 | 13 | 0.0085 | 2.4 |
| 25 | Italy - Sicily | 0.007 | 11 | 0.0008 | 25 |
| 26 | Italy - Sicily | 0.007 | 18 | 0.0008 | 22 |
| 27 | Italy - Puglia | 0.288 | 12 | 0.0087 | 11 |
| 28 | Italy - Abruzzo | 0.233 | 15 | 0.0008 | 24 |
| 29 | Italy - Sicily | 0.326 | 15 | 0.0008 | 16 |
| 30 | Italy - Sicily | 0.007 | 10 | 0.0087 | 10 |
| 31 | Italy - Sicily | 0.007 | 17 | 0.0008 | 21 |
| 32 | Italy - Sicily | 0.007 | 14 | 0.0008 | 22 |
| 33 | Italy - Sicily | 0.007 | 15 | 0.015 | 0.88 |
| 34 | Italy - Sicily | 0.007 | 16 | 0.0008 | 24 |
| 35 | Italy - Sicily | 0.007 | 18 | 0.016 | 7.6 |
| 36 | Italy - Sicily | 0.007 | 25 | 0.016 | 7.2 |
| 37 | Italy - Campania | 0.16 | 14 | 0.0008 | 14 |
| 38 | Italy - Sicily | 23 | 1.2 | 0.0008 | 33 |

| Place | Pt (µg/Kg) | Rh (µg/Kg) | Sample |
| --- | --- | --- | --- |
| Stuttgart | 2.9 4.6 | - - | roadside grass (1993) roadside grass (0.2 meters) (1994) |
| Gent (Belgio) | 1.4-1.7 | - | roadside grass |
| Germania | 3.61 10.6 ≤ 0.03 | 0.65 1.54 ≤ 0.03 | roadside grass (1994) roadside grass (1997) Area uncontaminated (1997) |
| Sheffield | 0.07-5.4 | - | Bark |
| Bialystok (Polonia) | 8.63 | 0.65 | roadside grass (1 meters) |
| San Francisco | 38 | - | Bark |

Table 4 - Platinum and rhodium concentrations in environmental matrices [29]

Table 5 Contamination categories based on EF values.

| EF < 2 | Deficiency to minimal enrichment |
| --- | --- |
| EF 2–5 | Moderate enrichment |
| EF 5–20 | Significant enrichment |
| EF 20–40 | Very high enrichment |
| EF N 40 | Extremely high enrichment |

Table 6- Geoaccumulation classes

| Class | Index | Significance |
| --- | --- | --- |
| 0 | < 0 | Practically uncontaminated |
| 1 | 0-1 | Uncontaminated to moderately contaminated |
| 2 | 1-2 | Moderately contaminated |
| 3 | 2-3 | Moderately to heavily contaminated |
| 4 | 3-4 | Heavily contaminated |
| 5 | 4-5 | Heavily to extremely contaminated |
| 6 | 5 | Extremely contaminated |