Tables Captions

Table 1 The operating conditions optimized for Cr(VI) determination with DP-ASV using a Aufilm-GCE sensor

| Experimental conditions: (DP-ASV) | Optimized Value |
| --- | --- |
| Supporting electrolyte | HCl 0.1 M |
| pH | 1.0 |
| Deposition time | 120 s |
| Deposition potential | -0.2 V |
| Detection potentiala | +0.3 V |
| Scan rate | 100 mV/s |
| Potential scan | -0.2 to 0.8 V |
| Modulation amplitude | 20 mV |
| Modulation time | 5 ms |

Table 2 Analytical performance of different modified electrodes for Cr(VI) determination.

| Modified electrode |  |  |  |  | Method | LOD (µg/L-1) |  |  |  | LDR (µg/L-1) |  |  | Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electrochemically activated GCE |  |  |  | DPV |  | 6.25 |  |  |  | 20.8–13000 |  |  | Richtera et al 2016 |  |  |  |  |
| Gold screen printed macro electrode |  |  |  | LSV |  | 228 |  |  |  | 520-84240 |  |  | Metters et al 2012 |  |  |  |  |
| AuNP-ITO electrode d |  |  |  | CV c |  | 104 | 260–5200 |  |  |  |  | Tsai and Chen 2008 |  |  |  |  |  |
|  |  |  |  | Amperometry |  | 5.2 | 26–2600 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Tsai and Chen 2008 |  |  |  |  |
| Ag NPs a - carbon SPE e |  |  |  | DPV |  | 44.2 |  | 26–1976 |  |  |  |  | Domínguez-Renedo et al 2008 |  |  |  |  |  |
| Au NPs - carbon SPE |  |  |  | DPV |  | 20.8 |  | 20.8–1664 |  |  |  |  |  |  |  |  |  |
| Ag plated-GCE |  |  |  | DP-ASV |  | 5.2 |  | 18.2-2080 |  |  | Stojanović et al 2018 |  |  |  |  |  |  |
| Graphite screen printed electrode |  |  |  | LSV |  | 18.7 |  | 99.8-998.4 |  |  |  |  |  |  |  | Hallam et al 2010 |  |
| Au film-GCE |  |  |  | DP-ASV |  | 5.5 |  |  | 10–120 |  |  |  |  |  | This work |  |  |
| aNPs – nanoparticles, bLSV – Linear sweep voltammetry, cCV – Cyclic voltammetry, dAuNP-ITO electrode – Gold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nanoparticle- electrodeposited indium-tin oxide electrode, e SPE – screen printed electrode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table. 3 Recovery test with sewage water samples

| Sample (sewage water) (µgL-1) | Cr(VI) Added (µgL-1) | Cr (VI) Found (µgL-1) | Spike Recovery (%) |
| --- | --- | --- | --- |
| Chromium Concentration | 88 | 86.1± 0.8 | 97.8 |
| 150 ± 0.6 | 176 | 176.5 ±0.5 | 100.3 |

Figure Captions

Fig. 1 Effect of the accumulation time (30s; 60s;120s; 180s) at Aufilm/GCE, in 0.1 M HCl for concentration 50 µg L-1 Cr(VI)

Fig. 2 a) Anodic stripping voltammograms of 50 µg L–1 Cr(VI) at various electrolyte. b) Effect of pH on the peak current for the determination of Cr(VI) on Aufilm-GCE

Fig. 3 a) DP-AS voltammograms registered with Aufilm/GCE electrode in HCl 0.1 M, tdep=120 s, poteciacial from -0.2 to 0.8 V in different concetration of Cr (VI) .b) Calibration curve of various Cr (VI) concentration

Fig. 4 DP-AS Voltammograms for standard additions of Cr(VI) in a sewage water sample, (0) sample signal before spiking and in concentrations (1)30, (2) 60, (3) 88 µg/L-1 Cr(VI)