## Supplementary information for: “Infrared thermography as an operando tool for the analysis of catalytic processes: how to use it?”

Robin Mutschler, Emanuele Moioli

Modelling of the reaction cell

The reaction cell is modeled by solving the mass and heat balances in a 1D pseudo-homogenous reactor model:

d(uci)dz= νiηρbr 										     (1)

uρbctotdTdz= νiηρbr-ΔHR+4dtubeUT(T-Te)						     (2)

The catalyst efficiency factor is calculated via the Thiele modulus:

ϕ=VpSpn+12⋅kci,sn-1D   		         							                   (3)

η=3ϕ2ϕcothϕ-1									     (4)

The heat transfer coefficient considers both the calculated by the contribution of convective and radiative heat transfer.

Convective heat transfer:

1UA=1ki+kclnrire+1ke										     (5)

k Is calculated considering a stagnant and a dynamic contribution:

k=k0+0.024⋅l⋅Redp									                   (6) The radiative heat transfer is calculated by linearization of the black body radiation equation, linearized in the temperature interval considered:

Qr=εσAT4-Te4=UB(T-Te)								     (7)

The reaction rate is calculated according to the model by Falbo et al. [1] for the 0.5% Ru/Al2O3 catalyst, while the reaction rate for the 2% Ru/Al2O3 catalyst is obtained by interpolation of the values proposed by Wang et al. [2]

List of symbols

u = gas velocity (m/s)

ci= concentration of the species i

νi= stoichiometric coefficient of the species i

η = catalyst efficiency

ρb= density of the bulk (kg/m3)

r = reaction rate (mol/s/m3)

ctot = total concentration (mol/m3)

T = Temperature (K)

ΔHR = reaction enthalpy (J/mol)

dtube = diameter of the tube (m)

UT = total heat transfer coefficient (W/m2/K)

UA = convective heat transfer coefficient (W/m2/K)

UB = radiative heat transfer coefficient (W/m2/K)

Te = temperature of the coolant (K)

ϕ = Thiele modulus

Vp = volume of the particle (m3)

S = surface of the particle (m2)

n = reaction order

D = diffusion coefficient (m2/s)

ki = local heat transfer coefficient (W/m2/K)

l = characteristic length (m)

Re = Reynolds number

ε = emissivity

σ = Stefan-Boltzmann constant (W/m2/K4)

References

1. 	Falbo, L.; Martinelli, M.; Visconti, C.G.; Lietti, L.; Bassano, C.; Deiana, P. Appl. Catal. B Environ. 2018, 225, 354–363, doi:10.1016/j.apcatb.2017.11.066.

2. 	Wang, X.; Hong, Y.; Shi, H.; Szanyi, J. J. Catal. 2016, 343, 185–195, doi:10.1016/j.jcat.2016.02.001.