Designing Microfluidic PCR Chip Device Using CFD Software for the Detection of Malaria

Meynard Austria 1, Jon Patrick Garcia 1, Alvin Caparanga 1, Lemmuel Tayo 1,2, and Bonifacio Doma 1,2

1School of Chemical, Biological, and Materials Engineering and Sciences, Mapúa University, Manila 1002, Philippines

2Department of Biology, School of Medicine and Health Sciences, Mapúa University, Makati 1200, Philippines

SUPPLEMENTARY MATERIAL

S1: Appendix A

Optimized Real-Time PCR Procedure for Malaria Detection in Low Level Parasite

Blood samples of 1 mL were drawn into sterile tubes containing EDTA. An aliquot of 500 μL were stored and used for DNA extraction. The DNA was extracted using500 μLTE buffer.

Low concentration of primer sequence of M60 and M61 and real time probe M62 were used as PCR mixture.  The PCR mixture contain 500 nM of each primer M60 (5’ACA TGG CTA TGA CGG GTA ACG3’) and M61 (5’TGC CTT CCT TAG ATG TGG TAG CTA3’) and 300 nM of M62 (5’FAMTM-TCA GGC TCC CTC TCC GGA ATC GA-TAMRATM3’). The thermal cycler was set at 95oC for 10s (denaturation), 58oC for 15s (annealing) and 72oC (extension) for 20s. The PCR mixture of 25 μL was added to 5 μL DNA sample.

S2: Appendix B

Viscosity Calculation

Falkenhagen Theory was used to evaluate the viscosity of PCR mixture and sample [1].

μsolutionμsolvent=1+ACsolute

μsolution(P) is the viscosity of the solution.

μsolvent(P) is the viscosity of the solvent.

Csolute(M) is the molar concentration of the solution.

A is a constant, dependent on the electrostatic forces of attraction.

A = 0.1365

For PCR mixture:

CPCR mix=CM60VPCRmix+CM61VPCRmix+CM62VPCRmixVPCRmix

CPCR mix=CM60+CM61+CM62VPCRmixVPCRmix

CPCR mix=CM60+CM61+CM62

CPCR mix=500+500+320 1000000000=1.32x10-6M

μPCR mixμWater=1+ACPCR mix

μPCR mix0.4665=1+0.13651.32x10-6M

μPCR mix0.4665 P=1+0.13651.32x10-6M

μPCR mix=0.46657 P

Results show that the viscosity of the PCR mixture is the same as the viscosity of the solvent water.

For sample concentration calculation:

CSample=CEDTAVEDTA+CTrisVTrisVsample

Csample=1.0 M x 5 mL+0.5M x 1mL500mL

Csample=0.011M

For Sample:

μsampleμwater=1+ACsample

μsample0.4665=1+0.13650.011

μsample=0.4731

To calculate for the viscosity of the solution (vsolution) containing the sample and PCR mixture REFUTAS equation was used.

vsolution= eeVBIsolution-10.97514.534-0.8

VBIsolution=wsample xVBITE+wPCR mix xVBIPCRmix

Where:

Wsample is the mass fraction of the sample in the solution.

WPCRmix is the mass fraction of PCR mixture in the solution.

VBI is the viscosity blending index calculated using the formula.

VBIsample=14.534 xlnlnvsample+0.8+10.975

VBIPCR mix=14.534 xlnlnvPCR mix+0.8+10.975

Where:

v is the kinematic viscosity which is the given by the equation below

v= μρ

ρsample =1.015

ρPCR mix=mM60+mM61+mM62+msolventVPCR mix

ρPCR mix=5x10-7Mx6574mgmmol+5x10-7Mx7429mgmmol+3x10-7M7049mgmmolx25mL+ 25mg25μl

ρPCR mix=1.009

Kinematic viscosity:

vsample= μsampleρsample=0.47311.015=0.4661 St

vPCR mix= μPCR mixρPCR mix=0.466571.009=0.4624 St

Viscosity Binding Index

VBIsample=14.534 xlnln0.4661+0.8+10.975=-10.0146

VBIPCR mix=14.534 xlnln0.4624+0.8+10.975=-10.1960

wsample= 1.015gmlx 5x 10-3ml1.015gmlx 5x 10-3ml+25.225 mg/1000=0.1675

wPCR mix=1-0.1675=0.8325

VBIsolution=0.1675 x-10.0146+0.8325 x-10.1960=-10.1657

vsolution= ee-10.1657-10.97514.534-0.8=0.4630 P

The viscosity of water at 60 oC is 0.467 P and the calculated viscosity of solution is 0.463 P. Since the viscosity difference is minimal, water properties were used in the simulation.

S3: Appendix C

Design Calculation

To calculate for the time consumed in each cycle:

t=VFRLD+ LE+2(LA-LEG+πrp+ 3 LE+4LS)πrc2+LCxWCxCN

tA=2LA-LEG+πrpV

tE=3LEV

tD=LDV

V=VFRACS

ACS=πrc2+LCxWC

Where:

t is the time consumed in second.

tAis the time consumed for annealing in second.

tEis the time consumed for annealing in second.

tDis the time consumed for annealing in second.

V is the velocity in micrometer per second.

VFR is the volumetric flowrate in cubic micrometer per second.

LD is the length of the denaturation copper plate in micrometer.

LE is the length of the extension copper plate in micrometer.

LA is the length of the annealing copper plate in micrometer.

LEG is the length of the edge gap of the fluid path to the edge of annealing copper plate in micrometer.

LS is the length of the space in micrometer.

rp is the radius of the path design in micrometer.

rc is the radius of the fluid path cross section in micrometer.

Lc and Wc are the length and width of the fluid path cross section in micrometer.

ACS is the cross-sectional area of the fluid path in square micrometer.

CNo is the number of cycles in the design.

Reference

[1]	X.D. Yang, R.V.N. Melnik, Effect of internal viscosity on Brownian dynamics of DNA molecules in shear flow, Comput Biol Chem. 31 (2007) 110–114. https://doi.org/10.1016/J.COMPBIOLCHEM.2007.02.010.