Table S1. Commercial microfluidic devices used to generate emulsion droplets.

| Company | Brand | Material | Characteristics | Applications | References |
| --- | --- | --- | --- | --- | --- |
| Dolomite | Telos ® 2 regent Telos ® 1 reagent 3D flow focusing SC | Glass microfluidic device | -The device consists of 7 parallel junctions that can be combined with others 10. -Hydrophilic, hydrophobic, and fluorophilic coating options. -Available at 100 or 50 µm. | Designed for emulsion generation, foam generation, micro-particle synthesis and high throughput experimentation (for example, analysis of cells). | (Dolomite Microfluidics, 2022) |
|  | Telos ® micromixer Chip |  | -The device consists of 7 independent micromixer channels on the chip (with 5 mixing stages each). -Hydrophilic and hydrophobic coating options. -Available at 30 or 50 µm. | Designed to create nano and micro particles and emulsions. |  |
|  | µEncapsulator 2 Reagent Droplet Chip |  | -Chip size 11.25mm x 15mm x 2mm. -Hydrophilic and fluorophilic coating surface options. -Junction of 100 µm scale. | Designed for rapid generation of double emulsion droplets (an aqueous core, surrounded by an oil shell) |  |
|  | µEncapsulator Sample Reservoir Chip |  | -Smooth channel with 100 µL storage volume. -Easy to clean and clear device. -Hydrophilic coating surface. | Designed for single emulsion droplet generation. |  |
|  | # Large Droplet Junction Chip | Glass and quartz device | -Chip offers both a T- and X-junction. -Simple and easy to use. -High droplet production (12,000 per second). -Hydrophilic, hydrophilic and, fluorophilic coating options. | Designed for droplet emulsion generation. |  |
|  | # Small Droplet Junction Chip |  | -Chip offers a flow focusing junction geometry with 14 x 17 µm cross-section at the junction. -High droplet production (12,000 per second). -Hydrophilic, hydrophilic and, fluorophilic coating options. | Designed for generating small droplets in the size range of 5-30 µm, improving control over the targeting and release of active compounds. |  |
|  | # T-Junction Chip |  | -Extremely smooth channel surface. - Excellent chemical compatibility. -Hydrophilic, hydrophilic and, fluorophilic coating options. | Designed for a range of applications, including mixing fluids, microreactions and droplet formation. |  |
|  | # 6-Junction Droplet Chip | Glass device | -The device consists of 6 separate flow-focusing junctions. -Excellent chemical compatibility. -Hydrophilic, hydrophilic and, fluorophilic coating options. | Designed for the parallel generation of 20 µm - 60 µm oil-in-water droplets. |  |
| Microfluidic ChipShop | # Fluidic 162 | Topas® COC (Cyclic olefin copolymer) or Polycarbonate PC | -One channel device. -Lid thickness 140 or 170 µm without surface treatment. | Designed for droplet generation on chip. | (microfluidic ChipShop, 2022) |
|  | # Fluidic 537 |  | -The flow-focusing device consists of 4 identical droplet generation units with a 38 µm nozzle size. |  |  |
|  | # Fluidic 912 |  | -The flow-focusing device consists of 8 identical droplet generation units with a channel dimension of 80 μm at the droplet formation region. |  |  |
|  | # Fluidic 536 |  | -The device consists of double-cross geometry with 37 µm nozzle size and 3 droplet generator units. | Designed for W/W/O double emulsion generation, allowing cell and particle encapsulation. |  |
|  | # Fluidic 488 |  | -The device consists of a combination of multiple T-junctions. -Storage module for capturing droplets. -Double cross nozzle type (74 µm). | Designed for generating single or multiple emulsions and storage. |  |
|  | # Fluidic 285 |  | -The multi-channel device presents various channels with different nozzle (50; 70; 80; 100 µm). -The main channel, as well as the entrance channel, vary in diameter, enabling a large set of experiments. | Designed for generating droplets with different volumes. |  |
|  | # Fluidic 1196 | Glass microfluidic device | -Several parallel microchannels in one unit. -Lid thickness of 250 µm -Channel dimensions 20 µm | Designed for droplet generation on chip. |  |

Table S2. Technological approaches and properties of the delivery systems based on food-grade emulsions assembled by microfluidic techniques.

| Emulsion type-Delivery system | Aqueous phase | Oil phase | Process conditions | Bioactive | Delivery system characterization | Microfluidic device type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| O/W- Emulsion | Modified lecithin (ML), whey protein isolate (WPI), or Tween 20 (0.05-3 wt%) | Medium chain triacylglycerol oil | Qd= 0.1-10 mL/h Qc= 100-500 mL/h | Fucoxanthin extract (0-4 wt%) | Size= 30.3 ± 0.2 μm (ML), 32.3 ± 0.1 μm (WPI) and 29.0 ± 0.8 μm (Tween 20) DI= 0.5 EE= 100%(ML) 90.7% (WPI) 50.4% (Tween 20) | Silicon 24 × 24 mm2 microchannels array chip (Model: WMS 11–1; EP. Tech Co., Ltd., Hitachi, Japan) | (Ma et al., 2020a) |
|  | Tween 20 (1% w/w), sodium salt of colic acid (Na-cholate), decaglycerol monolaurate, polyglyceryl-5-laurate (Sunsoft A-12E), or bovine serum albumin (BSA) (1% w/w) |  | Jd= 10-300 L/m.h | Quercetin (0.1-0.6 mg/mL) | Size= 28-29 μm DI < 0.21 EE= 80 % (4 ºC) and 70 % (25 ºC) | Silicon 24 × 24 mm2 microchannels array chip (Model WMS 1-2; EP. Tech Co., Ltd., Hitachi, Japan) | (Khalid et al., 2016a) |
|  | Decaglycerol monolaurate (ML-750) or Tween 20 (1% w/w) |  | Qd= 1-14 mL/h | β-sitosterol (0.5-4.0% w/w) and γ-oryzanol (0.5-4.0% w/w) | Size= 26-28 μm DI < 0.20 EE over 80 % | Silicon 24 × 24 mm2 microchannels array chip (Model: WMS 11–1; EP. Tech Co., Ltd., Hitachi, Japan) | (Khalid, Kobayashi, et al., 2017a) |
|  | Sodium dodecyl sulfate (SDS), decaglycerol monolaurate (ML-750), decaglycerol monooleate (MO-7S), sodium casinate (Na-Cs), and modified lecithin (ML) (1% w/w) |  | Qd= 0.25-5 mL/h | AstaReal® (AR, astaxanthin purity 20%), Zanthin® (ZA, astaxanthin purity 10%), and Astaxanthin >97% (Sigma- Aldrich) (1-5% w/w) | Size= 35-37 μm DI < 0.25 EE over 98 % |  | (Khalid, Shu, et al., 2017a) |
| O/W-Microgel | Sodium alginate (1 wt%), gelatin (5 wt%), and EDTA-Ca (2 wt%) | Tert-butyl hydroquinone (TBHQ) | Qd= 1-2 mL/h Qc= 10-20 mL/h | Vitamin A (weight ratio 4:1 vitamin A/TBHQ) | Size= 246 µm DI= 0.96% EE= not specified CR= 75% over 2h | Photosensitive resin microfluidic device made using microscale 3D printing equipment | (J. Zhang et al., 2022) |
|  | Poly (vinyl alcohol) (PVA) (0.25% w/v) | PLGA 7525A dissolved in dichloromethane (DCM) | - | Finasteride (28 mg, monthly doses) | Size around 30 µm DI= 23-28% EE= not specified CR= an initial burst, a moderate release, and then a plateau. | IVL-PPF Microsphere® microfluidic device | (J. H. Kim et al., 2019a) |
|  |  | PLGA 5050A (PURASORB® PDLG 5002A), PLGA 7525A (PURASORB® PDLG 7502A), and PLA02A (PURASORB® PDL 02A) dissolved in dichloromethane (DCM) | Pd= 1,100 mbar Pc= 2,200 mbar |  | Size= 40 μm (PLGA 7502A or 5002A) and around 30 µm (PLGA/PLA02A) DI= 0.28 and 0.16 EE > 96.5% |  | (J. H. Kim et al., 2021a) |
| O/W-Nanoparticle | D1: acetone or methanol D2: deionized water | 3M fluorinatedTM Fluid (FC-40) | Qc= 10 to 250 µL/min. Qd= fixed at 50 µL/min. | Itraconazole (ITZ) nanoparticles (1 mg/mL) | Size= the volume was calculated and range from 1 to 6 µL. EE= encapsulation was confirmed by the increase of particle size. | 316 stainless steel metal T-junction devices | (S. Kim et al., 2020) |
|  | Poly (vinyl alcohol) (PVA; 1% w/v) solution | PLGA (13.4 mg/mL) in acetonitrile (ACN) | Total flow rate: 2-12 mL/min Flow rate ratio= 2:1 to 10:1, aqueous:oilphases | Rutin (10 mg/mL) | Size= 123.4 nm DI= 0.16 ± 0.005 EE= 34 ± 2% | NanoAssemblr® Benchtop Device | (T. H. H. Vu et al., 2019) |
|  | Water | Tween 20 dissolved in ethyl acetate | Pc= 4.20 bar Pd= 3.81 bar For the smallest particle size | Fenofibrate (0.5 %wt) | Size < 1 µm DI= not specified | Glass microfluidic device | (Lorenz et al., 2018) |
| O/W/O- Core-shell microcapsule | Middle phase: an aqueous solution containing CaCl2 (100 × 10-3 M), disodium-EDTA (100 × 10-3 M) and sodium alginate (2.0%; pH 7.0) | Outer phase: soybean oil, acetic acid (5% w/v) and PGPR (5.0% w/v) Inner phase: soybean oil, benzyl benzoate (1:1), and PGPR (2.0% w/v) | Qi= 2.2 µL/h. Qm= 5.0 µL/h. Qo= 68.1 µL/h | Thyme essential oil and lavender essential oil (33.33% v/v) | Size= 182-342 µm DI= 2.4-2.8% EE= not specified | Glass microfluidic device | (Mou et al., 2020) |
| W/O/W- Giant liposome | Outer phase: poly (vinyl alcohol) (PVA; 10 % w/v) Inner phase: PVA (1% w/v) and dextran (9% w/v) | Middle phase: soybean lecithin (0.5% w/v) in the following organic solvent mixtures (1:1.8 v/v): chloroform/hexane; ethyl acetate/hexane or ethyl acetate/pentane. | Qi= 1000 μl/h Qm= 1000 μl/h Qo= 3,000-12,000 μl/h. | β-carotene (0.125% w/v) | Size= 100-180 µm DI= 3-6 % EE= not specified | Glass microfluidic device | (Michelon et al., 2019a) |
| W/O-Microgel | Pectin solution (0.5 and 1% w/w) | Acetic acid and CaCO3 in rapeseed oil | Qd= 1 mL/h Qc= 9 mL/h | Silver and gold nanoparticles (5.5 nm; 1.5 mg/mL) | The microgel was able to encapsulate nanoparticles | Polycarbonate (PC) microfluidic device | (Ogończyk et al., 2011) |
|  | Gellan gum (0.10–0.30% w/w) and Jabuticaba extract (diluted in 0.025 M potassium chloride pH 1 and 0.4 M sodium acetate pH 4.5) | PGPR (4% w/w) and calcium acetate (1% w/w) added to the soybean oil | Qd= 2-30 mL/h Qc= 150-250 mL/h | Jabuticaba extract (10-30% v/v) | Size= 185-342 µm DI= 0.016 and 0.086 | Glass microfluidic device | (Santos et al., 2020a) |
|  | 1: Dextran or protein aqueous solutions 2: Alginate solution (2% wt%) containing CaCO3 (200 mM). poly(ethyleneimine) (PEI; 0.3 wt%) or chitosan (1% v/v in acetic acid) coating | 3: Mineral oil with Span 80 (3 wt%), used as oil phase A 4: Mineral oil with Span 80 and acetic acid (0.2% v/v) used as oil phase B | Q1= 15 μL/min Q2= 0.5 μL/min Q3= 10 μL/min Q4= 14 μL/min | Ovalbumin | Size around 100 μm DI= not specified EE= 88% (PEI coating) and 80% (chitosan coating) | Polydimethylsiloxane (PDMS) microfluidic device (four inlets and one outlet) | (L. Yu et al., 2019b) |
|  | Alginate (2% w/v) and recombinant proteins in water Collection bath: chitosan (0.5%) and CaCl2 (0.1%) | Span 80 in mineral oil | Qd= 10 μl/h Qc= 50 μl/h | eGFP+AvrA nanoparticles (2% w/v) | Size= 339 ± 52 µm DI= 0.352 ± 0.153 EE= not specified CR= 70 % of the encapsulated was released after 240 min | Polydimethylsiloxane (PDMS) microfluidic device | (Ling et al., 2019a) |
|  | Pectin (1 wt%) Sheath flow: CaCl2 solution (1 wt%) | Mineral oil solution containing quercetin nanoparticles and retinyl palmitate, and Tween 80 (1 wt%) | Qd= 0.5 μL/min Qc= 4.0 μL/min Qsheath= 30 μL/min | Quercetin nanoparticles (0.02 wt%) and retinyl palmitate (0.2 wt%) | Size around 67.3-93.1 μm DI= not specified EE= not specified, encapsulation confirmed by fluorescence | Polydimethylsiloxane (PDMS) microfluidic device | (Noh et al., 2018a) |
| W/O/W- Emulsion | Inner phase: phosphate buffer solution (100 mmol/L) containing betanin and D-glucose (1% w/w) Outer phase: Tween 20 in water | Middle phase: soybean oil and tetraglycerin monolaurate condensed ricinoleic acid ester (CR-310) | Jd= 5-100 L/m.h | E162, (mixture of beetroot extract and maltodextrin; 0.4% w/w betanin, TCI/ABCr), betanin, and spray dried beetroot juice) (0.1-1.0% w/w) | Size < 50 μm DI < 0.26 ± 0.01 | Silicon 24 × 24 mm2 microchannels array chip (Model: WMS 11–1; EP. Tech Co., Ltd., Hitachi, Japan) | (Pagano et al., 2018a) |
| W/O/W- Solid lipid microparticle | Outer phase: water and polyvinyl alcohol (PVA; 10% (w/v) Inner phase: Ascorbic acid solution with or without CaCl2, and chitosan. | Middle phase: palm fat oil | Qi= 3,000 and 1,000 µL/h. Qm= 2,500 and 3,000 µL/h. Qo= 14,000 and 12,000 µL/h | Ascorbic acid (3-20% w/w) | Size= 195- 342 µm DI= 73-95 % EE= 73-95 % CR decreased around 50% after 30 days | Glass microfluidic device | (Comunian et al., 2014) |
| W/O/W- Microcapsule | Outer phase: aqueous solution and PVA Inner phase: aqueous mesoporous silica nanoparticles (MSN) + Rhodamine B (RB) solution | Middle phase: PLGA oil | Qi= 1 mL/h. Qm= 2 mL/h. Qo= 4 mL/h | Mesopouros silica nanoparticles (MSN; 40% w/v) + Rhodamine B (RB; 10-500 mg/L) | Size= 119 nm (MSN) and 56 µm (PLGA-MSN) DI= 4.91% EE around 88% for PLGA-MSN and 95% for PLGA CR around 96% after 120 days for PLGA-MSN and 97% after 120 days for PLGA | Polymethyl methacrylate (PMMA) microfluidic device | (Zhou et al., 2021) |
|  | Outer phase: Poly (vinyl alcohol) (PVA) aqueous solution (2 wt%) Inner phase: Poly (vinyl alcohol) (PVA) aqueous solution | Middle phase: PLGA (0.6 wt%) in dichloromethane) | Qi= 1000 μL/min Qm= 2000 μL/min Qo= 4000 μL/h | 2-[[(4-phenoxyphenyl)sulfonyl]methyl]-thiirane (SB-3CT; 0.5 mg/mL) | Size= 35-65 µm DI= 3% EE= 99% | Glass microfluidic device | (H. Chen et al., 2018a) |
|  | Middle phase: poly (N-isopropyl acry- lamide) (PNIPAM) | Inner phase: soybean oil solution added with camptothecin (0.4 mg/mL) Outer phase: soybean oil and PGPR | Qo= 1600-2400 µL/min Qm= 130-210 µL/min Qi= 20-100 µL/min | Doxorubicin hydrochloride and camptothecin (2 mg/mL) | Size= 600-1000 µm DI= not specified EE= not specified CR= burst process | Glass microfluidic device | (Z. Chen et al., 2022) |

*Qc: continuous phase flow rate; Qd: disperse phase flow rate; Qi: inner phase flow rate; Qm: middle phase flow rate; Qo: outer phase flow rate; Jd: disperse phase flux; Pd: disperse phase pressure; Pc: continuous phase pressure; Tween 20: polyoxyethylene (20) sorbitan monolaurate; EDTA-Ca: ethylenediaminetetraacetic acid calcium disodium salt hydrate; PGPR: polirricinoleato de poliglicerol; CaCl2: calcium chloride; CaCO3: calcium carbonate; PLGA: poly-lactic-co-glycolic acid; DI: dispersity index; EE: encapsulation efficiency; CR: compound release.

Table S3. Trends in droplet-based microfluidics approaches for fabricating delivery systems based on emulsions.

| Emulsion type | Continuous phase | Disperse phase | Surfactant | Microfluidic device type | Process conditions | Microfluidic device design | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| O/W emulsion | Aqueous surfactant solution | Palm oil | Span 20 | Combination of ultrasound and silicon/epoxy microchannel | Qd and Qc= 4 mL/min |  | (Manickam et al., 2020) |
|  | Xanthan gum or poloxamer in water | Sunflower oil | Tween 80 or Precirol™ | Microsystem at High Throughput (MHT) | Qd= 20.8-95.2 mL/min Qc= 500 mL/min. |  | (Nehme et al., 2021) |
|  | Argan by-products extract | Soybean oil | Tween 80 | Silicon | Qd= 2 mL/h |  | (Taarji et al., 2020) |
|  | Mineral oil and glutaraldehyde (5 %wt) | Chitosan (2 %wt) in hydrochloric acid (1 %wt) | Span 80 (3 %wt) | PDMS microfluidic device | Qd and Qc = 100 to 500 µL/h |  | (C. M. Kim et al., 2020) |
|  | Silicon oil and surfactant | Deionized water | Span 80 (1% w/v) | Photosensitive resin Gr |  |  | (J. Zhang, Xu, et al., 2021) |
|  | Water and PEG 2000 | Sunflower oil | Tween 20 | Glass microfluidic device | Qd and Qc= 20 to 280 µL/min | - | (Jurinjak Tušek et al., 2022) |
| W/O emulsion | Sunflower oil and surfactant | Whey protein isolated (WPI) | PGPR | Glass microfluidic device | Pressure ranged from 0-200 mbar for both phases |  | (Lacroix et al., 2022) |
|  | Fluorocarbon oil (HFE 7500) and surfactant | PPGDA:Dextran (20% w/w) Dextran and PPGDA:PPGA (1:3 in 300 µL of ethanol) | Fluorosurfactant 008 (2% w/w) | PDMS microfluidic device | Qd= 60 µL/h for PPGDA and 30 µL/h for dextran |  | (Keller et al., 2021) |
|  | Deionized water and surfactant | Mixture of mineral oil and heptane (50:50). | PVA (2% w/w). | Photosensitive resin Gr | Qd and Qc= 100 to 500 µL/h |  | (J. Zhang, Xu, et al., 2021) |
| W/O/W emulsion | Outer phase: glycerol (5% w/v) | Inner phase: glycerol (50% w/v) in water Middle phase: middle chain triglyceride (MCT) | PGPR in MCT | Glass microfluidic device | Qi= 1-5 mL/h Qm= 4-14 mL/h Qo= 50-400 mL/h |  | (Leister et al., 2022) |
|  | Outer phase: glycerol (25% w/w) in distilled water | Inner phase: Na-alginate (1.5 %wt) Middle phase: silicon oil | Span 80 (0.25% w/w) in middle phase. PVA (5% w/w) in outer phase | PDMS microfluidic device | Qi = 50 µL/h. Qm= 500 µL/h Qo=: 20000 - 100000 µL/h |  | (Sattari et al., 2021) |
|  | Outer phase: aqueous surfactant solution | Middle phase: phenylmethyl silicone oil and silicone resin RSN-0749 Inner phase: glycerol, PVA, and water | PVA (5 wt%) in outer and inner phases. | Glass microfluidic device | Qi= 150 mL/h Qm= 150-1750 mL/h Qo= 1 mL/h |  | (K. Zhang et al., 2022) |
|  | Outer phase: glycerol (40 wt%) and surfactant | Inner phase: glycerol (5 %wt) and surfactant Middle phase: XIA-METERÒ RSN-0749 resin in Dow Corning ® 200 fluid (2 wt%) | Hydrophilic: PVA Lipophilic: XIAMETER® RSN-074 | Glass microfluidic device |  |  | (Bandulasena et al., 2019) |
| O/W/O emulsion | Outer phase 1 and 2: oil solution containing PGPR (4% w/v) | Inner phase: poly(ethylene glycol) diacrylate (PEGDA; 50 %wt), 2-Hydroxy-methylpropiophenone (HMPP; 5 %wt), and surfactant | Pluronic ® F 127 | Glass microfluidic device | Qi= 100-900 µL/h Qo1= 300 and 200 µL/h Qo2= 500-800 µL/h |  | (Cai et al., 2019) |
|  | Outer phase: dimethicone | Inner phase: silicone oil. Middle phase: surfactant | PVA (2 % w/w) in the middle phase | Photosensitive resin Gr and HIT | Qi= Qm= 2 mL/h Qc= 5 mL/h |  | (J. Zhang, Xu, et al., 2021) |
|  | Outer phase: sunflower oil and glacial acetic acid (0.1% w/w) | O/W emulsion prepared by ultrasound: Na-alginate (1% (w/w)) and CaCO3 (0.1825 g/g of Na-alginate), cellulose nanocrystal and sunflower oil | PGPR (4 wt%) | Glass microfluidic device | Qc= 3000 - 15000 µL/h. Qd= 600-3000 µL/h |  | (Dias Meirelles et al., 2022) |
|  | Outer phase: silicone oil | Inner phase: silicone oil. Middle phase: glycerol (25% (w/w)) | Span 80 (0.25% w/w) in the inner phase. PVA (10 % w/w) and in the middle phase. Span 80 (2.5% w/w) in the outer phase | PDMS microfluidic device | Qi= 50 µL/h. Qm= 500 µL/h Qo= 20,000 - 100,000 µL/h |  | (Sattari et al., 2021) |
|  | Outer phase: orange oil | Inner phase: middle chain triglyceride (MCT). Middle phase: PVA (1% w/v) in water | PVA in water and PGPR in MCT | Glass microfluidic device | Qo=50-400 mL/h Qm= 4-14 mL/h Qi=1-5 mL/h |  | (Leister et al., 2022) |

* PVA: polyvinyl alcohol; PGPR: polyglycerol polyricinoleate; Span 20: sorbitan monooleate; PDMS: polydimethylsiloxane.