SUPPLEMENTARY MATERIAL

Table Captions

Table S1. Input and output data for process of biodiesel production.

Table S2. Input and output data for alcohol recycle unit.

Table S3. Input and output data for etherification reaction using ethanol and isopropanol at 110ºC and molar ratio of 1:6 (glycerol/alcohol).

Table S4. Input and output data from DWSIM for etherification reaction using ethanol and isopropanol at 110ºC and molar ratio of 1:12 (glycerol/alcohol).

Table S5. Input and output data for ether purification process (etherification using glycerol/ethanol molar ratio of 1:3 at 110ºC).

Table S6. Input and output data for ether purification process (etherification using glycerol/isopropanol molar ratio of 1:3 at 110ºC).

Table S1

| Conditions | Input |  | Output |  |
| --- | --- | --- | --- | --- |
|  | STR-01 |  | Biodiesel | Crude glycerol |
| Temperature (ºC) | 28.7 |  | 60.0 | 60.0 |
| Pressure (bar) | 1.01 |  | 1.01 | 1.01 |
| Mass Flow (kg/h) | 1227.12 |  | 1004.31 | 222.81 |
| Volumetric Flow (m3/h) | 1.347 |  | 1.178 | 0.260 |
| Specific Enthalpy (kJ/kg) | -203.64 |  | -232.52 | -936.48 |
| Component mole fraction |  |  |  |  |
| Triolein | 0.138 |  | 0.000 | 0.017 |
| Methanol | 0.830 |  | 0.436 | 0.728 |
| Sodium Hydroxide | 0.030 |  | 0.030 | 0.051 |
| Biodiesel | 0.000 |  | 0.394 | 0.000 |
| Crude glycerol | 0.000 |  | 0.131 | 0.219 |

STR-01: Mixture of vegetable oil, methanol and sodium hydroxide.

Table S2

| Alcohol recovery / Ethanol |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conditions | Input |  |  | Output |  |  |  |
|  | Polar comp. |  |  | STR-67 |  | Alcohol |  |
| Temperature (ºC) | 79.9 |  |  | -10.0 |  | -10.0 |  |
| Pressure (bar) | 1.01 |  |  | 1.01 |  | 1.01 |  |
| Mass Flow (kg/h) | 117.89 |  |  | 51.00 |  | 66.89 |  |
| Volumetric Flow (m3/h) | 0.137 |  |  | 0.050 |  | 0.082 |  |
| Spec. Enthalpy (kJ/kg) | -1162.72 |  |  | -1931.98 |  | -1012.28 |  |
| Component mole fraction |  |  |  |  |  |  |  |
| Ethanol | 0.434 |  |  | 0.000 |  | 1.000 |  |
| Glycerol | 0.068 |  |  | 0.121 |  | 0.000 |  |
| Water | 0.496 |  |  | 0.878 |  | 0.000 |  |
| Alcohol recovery / Isopropanol |  |  |  |  |  |  |  |
| Conditions |  | Input |  |  | Output |  |  |  |
|  |  | Polar comp. |  |  | STR-67 |  | Alcohol |  |
| Temperature (ºC) |  | 85.8 |  |  | -10.0 |  | -10.0 |  |
| Pressure (bar) |  | 1.01 |  |  | 1.01 |  | 1.01 |  |
| Mass Flow (kg/h) |  | 203.86 |  |  | 52.78 |  | 151.07 |  |
| Volumetric Flow (m3/h) |  | 26.267 |  |  | 0.043 |  | 0.186 |  |
| Spec. Enthalpy (kJ/kg) |  | -527.98 |  |  | -1523.60 |  | -844.97 |  |
| Component mole fraction |  |  |  |  |  |  |  |
| Isopropanol |  | 0.704 |  |  | 0.000 |  | 1.000 |  |
| Glycerol |  | 0.127 |  |  | 0.433 |  | 0.000 |  |
| Water |  | 0.167 |  |  | 0.566 |  | 0.000 |  |

STR-67: Mixture of water and glycerol.

Table S3

| Etherification with ethanol (1:6 molar ratio) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conditions | Input |  |  |  |  | Output |  |  |
|  | Glycerol |  | Ethanol |  |  | STR-12 | STR-13 |  |
| Temperature (ºC) | 229.2 |  | 25.0 |  |  | 110.0 | 110.0 |  |
| Pressure (bar) | 1.01 |  | 1.01 |  |  | 1.01 | 1.01 |  |
| Mass Flow (kg/h) | 95.51 |  | 286.66 |  |  | 219.59 | 162.57 |  |
| Volumetric Flow (m3/h) | 0.087 |  | 0.364 |  |  | 166.250 | 0.188 |  |
| Spec. Enthalpy (kJ/kg) | -475.76 |  | -922.64 |  |  | 134.65 | -587.50 |  |
| Component mole fraction |  |  |  |  |  |  |  |  |
| Ethanol | 0.000 |  | 1.000 |  |  | 0.829 | 0.259 |  |
| Glycerol | 1.000 |  | 0.000 |  |  | 0.000 | 0.112 |  |
| 3-ethoxypropan-1,2-diol | 0.000 |  | 0.000 |  |  | 0.000 | 0.154 |  |
| 1,3-diethoxypropan-2-ol | 0.000 |  | 0.000 |  |  | 0.002 | 0.255 |  |
| 1,2,3-triethoxypropane | 0.000 |  | 0.000 |  |  | 0.000 | 0.000 |  |
| Water | 0.000 |  | 0.000 |  |  | 0.169 | 0.220 |  |
| Etherification with isopropanol (1:6 molar ratio) |  |  |  |  |  |  |  |  |
| Conditions |  | Input |  |  |  | Output |  |  |
|  |  | Glycerol | Isopropanol |  |  | STR-12 |  | STR-13 |
| Temperature (ºC) |  | 229.2 | 25.0 |  |  | 110.0 |  | 110.0 |
| Pressure (bar) |  | 1.01 | 1.01 |  |  | 1.01 |  | 1.01 |
| Mass Flow (kg/h) |  | 95.51 | 373.94 |  |  | 313.26 |  | 156.19 |
| Volumetric Flow (m3/h) |  | 0.087 | 0.478 |  |  | 176.982 |  | 0.174 |
| Spec. Enthalpy (kJ/kg) |  | -475.76 | -754.44 |  |  | 140.72 |  | -615.86 |
| Component mole fraction |  |  |  |  |  |  |  |  |
| Isopropanol |  | 0.000 | 1.000 |  |  | 0.894 |  | 0.309 |
| Glycerol |  | 1.000 | 0.000 |  |  | 0.000 |  | 0.224 |
| 3-isopropoxypropan-1,2-diol |  | 0.000 | 0.000 |  |  | 0.000 |  | 0.402 |
| 1,3-di-isopropoxypropan-2-ol |  | 0.000 | 0.000 |  |  | 0.000 |  | 0.010 |
| Water |  | 0.000 | 0.000 |  |  | 0.106 |  | 0.055 |

STR-12: Reaction mixture (Vapor phase); STR-13: Reaction mixture (Liquid phase).

Table S4

| Etherification with ethanol (1:12 molar ratio) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conditions | Input |  |  |  |  | Output |  |  |
|  | Glycerol |  | Ethanol |  |  | STR-12 | STR-13 |  |
| Temperature (ºC) | 229.2 |  | 25.0 |  |  | 110.0 | 110.0 |  |
| Pressure (bar) | 1.01 |  | 1.01 |  |  | 1.01 | 1.01 |  |
| Mass Flow (kg/h) | 95.51 |  | 573.32 |  |  | 495.01 | 173.82 |  |
| Volumetric Flow (m3/h) | 0.087 |  | 0.729 |  |  | 365.001 | 0.196 |  |
| Spec. Enthalpy (kJ/kg) | -475.76 |  | -922.64 |  |  | 134.14 | -465.04 |  |
| Component mole fraction |  |  |  |  |  |  |  |  |
| Ethanol | 0.000 |  | 1.000 |  |  | 0.860 | 0.281 |  |
| Glycerol | 1.000 |  | 0.000 |  |  | 0.000 | 0.092 |  |
| 3-ethoxypropan-1,2-diol | 0.000 |  | 0.000 |  |  | 0.000 | 0.089 |  |
| 1,3-diethoxypropan-2-ol | 0.000 |  | 0.000 |  |  | 0.001 | 0.167 |  |
| 1,2,3-triethoxypropane | 0.000 |  | 0.000 |  |  | 0.002 | 0.185 |  |
| Water | 0.000 |  | 0.000 |  |  | 0.136 | 0.186 |  |
| Etherification with isopropanol (1:12 molar ratio) |  |  |  |  |  |  |  |  |
| Conditions |  | Input |  |  |  | Output |  |  |
|  |  | Glycerol | Isopropanol |  |  | STR-12 |  | STR-13 |
| Temperature (ºC) |  | 229.2 | 25.0 |  |  | 110.0 |  | 110.0 |
| Pressure (bar) |  | 1.01 | 1.01 |  |  | 1.01 |  | 1.01 |
| Mass Flow (kg/h) |  | 95.51 | 745.35 |  |  | 676.14 |  | 164.72 |
| Volumetric Flow (m3/h) |  | 0.087 | 0.953 |  |  | 371.136 |  | 0.188 |
| Spec. Enthalpy (kJ/kg) |  | -475.76 | -754.44 |  |  | 140.46 |  | -588.77 |
| Component mole fraction |  |  |  |  |  |  |  |  |
| Isopropanol |  | 0.000 | 1.000 |  |  | 0.933 |  | 0.335 |
| Glycerol |  | 1.000 | 0.000 |  |  | 0.000 |  | 0.128 |
| 3-isopropoxypropan-1,2-diol |  | 0.000 | 0.000 |  |  | 0.000 |  | 0.494 |
| 1,3-di-isopropoxypropan-2-ol |  | 0.000 | 0.000 |  |  | 0.000 |  | 0.011 |
| Water |  | 0.000 | 0.000 |  |  | 0.067 |  | 0.032 |

STR-12: Reaction mixture (Vapor phase); STR-13: Reaction mixture (Liquid phase).

Table S5

| Ether purification for simulation using ethanol (1:3 molar ratio) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conditions | Input |  |  |  | Output |  |  |
|  | STR-12 | STR-13 | Solvent |  | Ethers | Solvent | Polar Comp. |
| Temperature (ºC) | 110.0 | 110.0 | 25.0 |  | 213.9 | 69.8 | 79.8 |
| Pressure (bar) | 1.01 | 1.01 | 1.01 |  | 1.01 | 1.01 | 1.01 |
| Mass Flow (kg/h) | 59.62 | 179.21 | 238.83 |  | 116.12 | 243.66 | 117.88 |
| Molar Flow (kmol/h) | 1.65 | 2.50 | 2.77 |  | 0.78 | 2.80 | 3.34 |
| Vol. Flow (m3/h) | 51.91 | 0.20 | 0.36 |  | 0.14 | 53.65 | 0.14 |
| Spec. Enthalpy (kJ/kmol) | 137.42 | -614.17 | -366.01 |  | -92.81 | -30.33 | -1173.78 |
| Component mole fraction |  |  |  |  |  |  |  |
| Ethanol | 0.636 | 0.160 | 0.000 |  | 0.000 | 0.000 | 0.434 |
| Glycerol | 0.000 | 0.092 | 0.000 |  | 0.000 | 0.000 | 0.068 |
| 3-ethoxypropan-1,2-diol | 0.000 | 0.095 | 0.000 |  | 0.304 | 0.000 | 0.000 |
| 1,3-dietoxipropan-2-ol | 0.001 | 0.115 | 0.000 |  | 0.361 | 0.001 | 0.000 |
| 1,2,3-triethoxypropane | 0.001 | 0.112 | 0.000 |  | 0.324 | 0.010 | 0.000 |
| Water | 0.361 | 0.425 | 0.000 |  | 0.000 | 0.000 | 0.496 |
| N-hexane | 0.000 | 0.000 | 1.000 |  | 0.010 | 0.988 | 0.000 |

STR-12: Reaction mixture (Vapor phase); STR-13: Reaction mixture (Liquid phase).

Table S6

| Ether purification for simulation using isopropanol (1:3 molar ratio) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conditions | Input |  |  |  | Output |  |  |
|  | STR-12 | STR-13 | Hexane |  | Ethers | Hexane | Polar Comp. |
| Temperature (ºC) | 110.00 | 110.00 | 25.00 |  | 224.30 | 69.09 | 81.93 |
| Pressure (bar) | 1.01 | 1.01 | 1.01 |  | 1.01 | 1.01 | 1.01 |
| Mass Flow (kg/h) | 675.91 | 164.72 | 840.63 |  | 97.85 | 854.59 | 728.83 |
| Molar Flow (kmol/h) | 11.80 | 1.63 | 9.75 |  | 0.73 | 9.85 | 12.61 |
| Vol. Flow (m3/h) | 371.02 | 0.19 | 1.28 |  | 0.12 | 1.38 | 0.98 |
| Spec. Enthalpy (kJ/kmol) | 140.44 | -587.04 | -366.01 |  | -277.79 | -261.67 | -615.80 |
| Component mole fraction |  |  |  |  |  |  |  |
| Isopropanol | 0.932 | 0.335 | 0.000 |  | 0.000 | 0.000 | 0.916 |
| Glycerol | 0.000 | 0.127 | 0.000 |  | 0.000 | 0.000 | 0.016 |
| 3-ethoxypropan-1,2-diol | 0.000 | 0.494 | 0.000 |  | 0.975 | 0.010 | 0.000 |
| 1,3-dietoxipropan-2-ol | 0.000 | 0.011 | 0.000 |  | 0.014 | 0.001 | 0.000 |
| Water | 0.067 | 0.031 | 0.000 |  | 0.000 | 0.000 | 0.067 |
| N-hexane | 0.000 | 0.000 | 1.000 |  | 0.010 | 0.989 | 0.000 |

STR-12: Reaction mixture (Vapor phase); STR-13: Reaction mixture (Liquid phase).