Supplementary Materials

Table S1. Properties of the feedstocks.

| Properties | Oat straw | Sawdust | Source |
| --- | --- | --- | --- |
| Bulk density (kg/m3) | 127.87 | 158.00 | [29,31] |
| HHV (MJ/kg) | 16.84 | 18.02 | [29,31,34] |
| Cellulose (%) | 47.45 | 41.24 | [29,31] |
| Hemicellulose (%) | 24.70 | 15.72 | [29,31,34] |
| Lignin (%) | 12.92 | 25.16 | [29,31] |
| Ash (%) | 5.32 | 0.54 | [29,31] |
| Other solids (%) | 9.61 | 7.34 | [29,31] |
| Moisture content (%) | 10 | 10 | [29,31] |

NB; HHV: Higher heating value

Table S2. Cost assumptions and economic evaluation parameters.

| Parameters | Value | Source |
| --- | --- | --- |
| Time parameters |  |  |
| Year of analysis | 2023 |  |
| Currency | US Dollars ($) |  |
| Construction period | 12 months |  |
| Startup period | 6 months |  |
| Project lifetime | 20 years |  |
| Inflation | 2% |  |
| Financial parameters |  |  |
| Depreciation style | Straight line |  |
| Depreciation time | 10 years |  |
| Revenue tax rate | 40% |  |
| Salvage value | 5% of DFC |  |
| Discount rate | 7% |  |
| Operating cost |  |  |
| Operation capacity | 90% |  |
| Labor | $ 69/h | SuperPro designer databank |
| Sawdust | $ 75/t | Antonio et al. [40] |
| Oat straw | $ 70/t | BIMAT [36] |
| Nitrogen gas | 0.11 / Nm3 | EIA [6] |
| Selling price of pellet | $60/t - $260/t | [18,26,51] |
| Selling price of torrefaction liquid | $0.25/L | Badger et al. [27] |
| Utilities |  |  |
| Steam (High P) | $ 20/t | SuperPro designer databank |
| Steam | $ 12/t | SuperPro designer databank |
| Cooling water | $ 0.05/t | SuperPro designer databank |
| Electricity | $ 0.1/kW-h | SuperPro designer databank |

NB; DFC: Direct fixed capital cost.

Figure S1. The process of pelleting raw sawdust (Case scenario 1).

Figure S2. The process of pelleting raw oat straw (Case scenario 2).

Figure S3. Integrated torrefied treated oat straw pelletization plant utilizing torrefaction liquid (TL) as binder (Case scenario 4).

Figure S4. Integrated steam exploded treated sawdust pellet plant (case scenario 5).

Table S3. Major equipment costs (2023 prices in US$) for scenarios 1 and 2.

| Description | Cost ($) (number of units in parentheses) |  |
| --- | --- | --- |
|  | Scenario 1 | Scenario 2 |
| Silo/bin | 477,000 (4) | 247,000 (2) |
| Grinder |  | 154,000 (1) |
| Screw conveyor | 176,000 (2) | 162,000 (2) |
| Belt conveyor |  | 177,000 (1) |
| Pelletizer | 212,000 (1) | 212,000 (1) |
| Heat exchanger | 23,000 (1) | 23,000 (1) |
| Bucket elevator | 20,000 (1) | 20,000 (1) |
| Unlisted equipment | 230,000 | 265,000 |
| Total | 1,148,000 | 1,323,000 |

Table S4. Major equipment costs (2023 prices in US$) for scenarios 3 and 4.

| Description | Cost ($) (number of units in parentheses) |  |
| --- | --- | --- |
|  | Scenario 3 | Scenario 4 |
| Silo/bin | 477,000 (4) | 338,000 (3) |
| Grinder |  | 301,000 (2) |
| Screw conveyor | 374,000 (4) | 254,000 (3) |
| Torrefaction reactor | 3,875.000 (1) | 3,875.000 (1) |
| Condenser | 34,000 (1) | 34,000 (1) |
| Flow splitter | 163,000 (1) | 163,000 (1) |
| Horizontal tank | 48,000 (2) | 24,000 (1) |
| Belt conveyor |  | 789,000 (3) |
| Pelletizer | 185,000 (1) | 212,000 (1) |
| Vibrating screen | 33,000 (1) | 38,000 (1) |
| Heat exchanger | 420,000 (8) | 236,000 (4) |
| Bucket elevator | 20,000 (1) | 23,000 (1) |
| Unlisted equipment | 1,410,000 | 1,572,000 |
| Total | 7,048,000 | 7,859,000 |

Table S5. Major equipment costs (2023 prices in US$) for scenarios 5 and 6.

| Description | Cost ($) (number of units in parentheses) |  |
| --- | --- | --- |
|  | Scenario 5 | Scenario 6 |
| Steam generator | 638,000 (1) | 641,000 (1) |
| Dryer | 131,000 (1) | 264,000 (2) |
| Silo/bin | 140,000 (2) | 70,000 (1) |
| Grinder |  | 109,000 (1) |
| Screw conveyor | 272,000 (3) | 275,000 (3) |
| Belt conveyor |  | 263,000 (3) |
| Pelletizer | 273,000 (1) | 273,000 (1) |
| Vibrating screen | 13,000 (1) | 12,000 (1) |
| Heat exchanger | 210,000 (4) | 363,000 (6) |
| Bucket elevator | 23,000 (1) | 23,000 (1) |
| Unlisted equipment | 425,000 | 535,000 |
| Total | 2,125,000 | 2,675,000 |