Table 1. Stages and resources taken into account when assessing the life cycle of technically complex devices

| Object | Source | Production |  |  |  |  |  | Installation |  | Operation |  | Waste management after end-of-life operation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Number of materials used, units | Types of processing | Auxiliary structures | Electricity | Other resources* | Transporta-tion of materials and components | Land acquisition, site preparation, road | Transportation of structures | Technical service | Repair |  |
| Offshore wind turbine (Brazil, 2022) | [20] | Total – 5 Gandola – 4 Hub and blades – 2 Tower – 3 Platform – 2 Mooring system - 1 | were not considered | were not considered | + | were not considered | Freight transport | were not considered | Sea transport | Technical service - 2 times / year, Lubricating oil -1 time / 3 years | In case of breakdown | Recycling |
| Offshore wind turbine (USA, 2014) | [21] | Total – 6 Gandola – 5 Blades – 4 Tower - 1 | were not considered | were not considered | - | Lubricating oil | were not considered | were not considered | Sea transport | were not considered | were not considered | Recycling, incineration, disposal |
| Onshore wind turbine (USA, 2014) | [22] | Total – 7 Gandola – 5 Rotor – 4 Tower - 1 | were not considered | Foundation | - | Lubricating oil | were not considered | were not considered | Freight transport | Changing oils and lubricants - 1 time / 3 years, rotor blades, reducer and generator 1 time / 20 years | were not considered | Recycling, incineration, disposal |
| Onshore wind turbine (USA, 2019) | [23] | Total – 12 Gandola – 9 Rotor – 9 Tower - 2 | were not considered | Foundation, electrical wiring, substation | + | Lubricating oil, varnish, glue, electronic components | no data | Site and access road preparation | Sea and freight transport | Lubricating oil -1 time / 2 years, brake system – 1 time / 5 years, control system - 1 time / 10 years | In case of breakdown | Recycling, incineration, disposal |
| Onshore wind turbine (USA, 2022) | [24] | Total – 9 Gandola – 5 Blades – 2 Tower - 1 | were not considered | Foundation | + | were not considered | were not considered | were not considered | Sea and freight transport | were not considered | were not considered | were not considered |
| Wind power plant (Jordan, 2019) | [25] | Total – 15 | no data | Foundation, electrical wiring, substation | + | Varnish, glue, sealant, electronic components | were not considered | Site and access road preparation | Sea and freight transport | Lubricating oil -1 time / 5 years | In case of breakdown | Recycling, incineration, disposal |
| Onshore wind turbine (Iran, 2018) | [26] | Total – 8 | no data | Foundation | + | were not considered | were not considered | were not considered | were not considered | Changing the oil in the gearbox and cooling system, lubrication of mechanical parts | were not considered | Recycling, incineration, disposal |
| Onshore wind turbine (Germany, 2010) | [13] | Total – 13 | Injection casting, rolling, wire drawing | Foundation, electrical wiring, substation | + | were not considered | were not considered | Site and access road preparation | were not considered | were not considered | were not considered | Recycling, disposal |
| Onshore wind turbine (France, 2003) | [13] | Total – 13 | Injection molding, rolling, wire drawing, polymerization | Foundation, electronics | + | Paint (alkyd) | were not considered | Land acquisition, site and access road preparation | were not considered | were not considered | were not considered | Recycling, disposal |
| Onshore wind turbine (Denmark, 2004) | [13] | Total – 12 | Injection casting, rolling, welding, wire drawing | Foundation, electronics | + | Zinc coating | were not considered | Land acquisition, site and access road preparation | were not considered | were not considered | were not considered | Recycling, disposal |
| Onshore wind turbine (World, 2014) | [27] | Total – 7 Blades – 2 Tower - 1 | were not considered | Electronics | - | Water, lubricating oil | were not considered | were not considered | Sea, railway, freight transport | Replacement of consumables | In case of breakdown | Recycling, incineration, disposal |
| Onshore wind turbine (EU, 2020) | [28] | Total – 12 Gandola – 7 Generator – 5 Gearbox - 5 Tower – 7 Blades - 1 | were not considered | Foundation, submarine cable | + | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered |
| Electric motor for electric vehicle (Switzerland, 2015) | [13] | Total – 10 | Rolling, wire drawing, coating | Electronics | + | Heat supply, nickel coating | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered |
| Electric vehicle (Canada, 2018) | [29] | Total – 11 | were not considered | were not considered | + | Paints, coatings | Sea and freight transport | were not considered | were not considered | were not considered | were not considered | Recycling |
| Electric vehicle (EU, 2022) | [30] | Total – 11 | were not considered | Electronics | + | Textile | were not considered | Road construction | were not considered | Replacement of tires, brake pads | were not considered | Recycling, disposal |
| Hydrogen fuel cell power plant for a bus (Russian Federation, 2021) | [31] | Total – 6 | no data | Fuel cell | + | were not considered | were not considered | were not considered | were not considered | Replacement of сonsumables | In case of breakdown | Recycling, disposal |
| Hydrogen fuel cell propulsion system for bus (Russian Federation, 2021) | [31] | Total – 5 | no data | Accumulator | + | were not considered | were not considered | were not considered | were not considered | Replacement of сonsumables | In case of breakdown | Recycling, disposal |
| Passenger plane (World, 2019) | [13] | Total – 5 | Injection molding | were not considered | + | Water, heat supply | were not considered | were not considered | were not considered | were not considered | were not considered | Recycling |
| Passenger plane (Switzerland, 2015) | [32] | Total – 5 | were not considered | were not considered | - | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered | Recycling, disposal |
| Passenger plane (UK, 2013) | [33] | Total (without aircraft engine) – 5 | no data | no data | + | no data | Air and road transport | were not considered | Air and road transport | were not considered | were not considered | Recycling, incineration, disposal |
| Aircraft engine (USA, 2015) | [34] | Total – 5 | were not considered | were not considered | - | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered |
| Marine engine (Peru,2018) | [13] | Total – 5 | Metalworking (steel) | were not considered | - | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered | Recycling |
| Bus (Germany, 2004) | [13] | Total – 13 | Rolling (steel), tempering (glass), wire drawing (copper) | Manufacture building | + | Heat supply, diesel, lubricating oil, acetic, hydrochloric, nitric, sulfuric acid, paint (alkyd), refrigerant, sodium hydroxide, etc. | were not considered | were not considered | were not considered | were not considered | were not considered | Recycling |
| Gas turbine power plant (Brazil, 2003) | [14] | Total – 4 | no data | Foundation | + | were not considered | Freight transport | were not considered | Freight transport | Replacement of consumables (oil, spare parts, etc.) | were not considered | Recycling, disposal |
| Gas turbine installation (EU, 2007) | [13] | Total – 5 | were not considered | Foundation | + | Heat supply | were not considered | Land acquisition | were not considered | were not considered | were not considered | were not considered |
| Gas turbine installation (Norway, 2021) | [13] | Total – 8 | Rolled sheets, metalworking (steel), injection molding (polymers)) | were not considered | - | Organic chemicals | were not considered | were not considered | were not considered | were not considered | were not considered | Incineration, disposal |
| Micro gas turbine installation (EU, 2007) | [13] | Total – 7 | Rolled sheets, welding, polymerization, construction work | Manufacture building | + | Heat supply, water | were not considered | Land acquisition | Road transport | were not considered | were not considered | were not considered |
| Combined cycle steam power plant (EU, 2007) | [13] | Total – 12 | were not considered | were not considered | + | Heat supply, diesel | were not considered | Land acquisition | were not considered | were not considered | were not considered | were not considered |
| Combined cycle steam power plant (Indonesia, 2021) | [15] | no data | were not considered | were not considered | + | Water, reagents, lubricating oil | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered |
| Combined cycle steam power plant (Indonesia, 2022) | [16] | no data | were not considered | no data | + | Water, reagents, lubricating oil, chemicals | no data | Land acquisition, site preparation | no data | Replacement of сonsumables | no data | Recycling, incineration, disposal |
| Combined cycle steam power plant (China, 2023) | [17] | Total – 2 | were not considered | were not considered | + | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered | were not considered |
| Combined cycle power plant (Italy, 2019) | [18] | Total – 15 | were not considered | were not considered | + | Water (salty, fresh) | were not considered | Land acquisition, site preparation | no data | Replacement of сonsumables | were not considered | Recycling, incineration, disposal |
| Thermal power plant (Russian Federation, 2021) | [19] | Total - 4 Engine – 3 Generator – 2 Container - 1 | were not considered | were not considered | + | Water | were not considered | were not considered | were not considered | were not considered | were not considered | Recycling |

* Water consumption, heat supply, fuel for construction machines, paint, lubricating oils, etc.