Tab. S1   ED XRF limits of quantification [ng/m3].

| Na | Mg | Al | Si | S | Cl | K | Ca | Ti | V | Cr | Mn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 73 | 89 | 0.46 | 1.1 | 21 | 22 | 10 | 18 | 1.6 | 1.3 | 15 | 5.0 |
| Fe | Ni | Cu | Zn | As | Se | Cd | In | Sb | Ba | Pb |  |
| 16 | 2.4 | 3.6 | 4.3 | 0.50 | 1.8 | 25 | 57 | 7.7 | 1.2 | 2.7 |  |

S2

Tab. S2  ED XRF mean relative uncertainties [%].

| Na | Mg | Al | Si | S | Cl | K | Ca | Ti | V | Cr | Mn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 3 | 10 | 5 | 1 | 2 | 1 | 1 | 3 | 49 | 22 | 15 |
| Fe | Ni | Cu | Zn | As | Se | Cd | In | Sb | Ba | Pb |  |
| 2 | 101 | 21 | 11 | 116 | 39 | 880 | 908 | 18 | 14 | 29 |  |

Tab. S3  Minimum, maximum, median and mean concentrations in PM10 and mean relative uncertainties. Min: minimum; Max: maximum; Med: median; Avg: mean; NA: no data available, LVG: levoglucosan, MAN: mannosan; GAL: galactosan. All units in ng/m3, except the PM10 (µg/m3), and relative uncertainty (unitless).

| Species | TOBA |  |  |  |  | TOKU |  |  |  |  | TORE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Min | Max | Med | Avg | Unc | Min | Max | Med | Avg | Unc | Min | Max | Med | Avg | Unc |
| OC | 1 827 | 41 557 | 6 955 | 8 940 | 11% | 1 827 | 51 719 | 6 008 | 8 131 | 13% | 1 827 | 48 093 | 7 917 | 10 034 | 9% |
| EC | 10 | 12 325 | 2 024 | 2 705 | 12% | 10 | 14 746 | 1 621 | 2 363 | 13% | 10 | 14 402 | 2 791 | 3 318 | 10% |
| OC1 | 290 | 10 472 | 1 035 | 1 624 | 12% | 353 | 14 109 | 1 017 | 1 581 | 15% | 144 | 13 315 | 1 667 | 2 299 | 9% |
| OC2 | 277 | 5 337 | 943 | 1 215 | 13% | 257 | 6 461 | 856 | 1 139 | 17% | 150 | 6 041 | 1 026 | 1 295 | 10% |
| OC3 | 446 | 5 758 | 1 309 | 1 565 | 15% | 434 | 6 662 | 1 179 | 1 420 | 19% | 349 | 9 620 | 1 295 | 1 539 | 10% |
| OC4 | 474 | 4 207 | 1 771 | 1 860 | 16% | 419 | 4 646 | 1 457 | 1 547 | 22% | 432 | 7 598 | 1 940 | 2 089 | 11% |
| EC1 | 219 | 14 228 | 1 730 | 2 668 | 11% | 214 | 23 328 | 1 545 | 2 501 | 12% | 75 | 19 354 | 1 905 | 2 779 | 11% |
| EC2 | 140 | 13 887 | 1 233 | 1 938 | 11% | 142 | 12 624 | 1 096 | 1 676 | 12% | 5 | 13 975 | 1 270 | 1 851 | 11% |
| EC3 | 89 | 2 012 | 455 | 540 | 13% | 137 | 1 775 | 402 | 539 | 14% | 38 | 3 482 | 647 | 827 | 12% |
| EC4 | 105 | 1 052 | 337 | 378 | 14% | 25 | 999 | 237 | 276 | 17% | 25 | 3 122 | 552 | 735 | 14% |
| LVG | 5 | 3 177 | 223 | 386 | 19% | 5 | 3 826 | 243 | 392 | 19% | 5 | 3 327 | 229 | 379 | 19% |
| MAN | 5 | 694 | 33 | 67 | 19% | 5 | 627 | 49 | 75 | 20% | 5 | 641 | 61 | 90 | 20% |
| GAL | 5 | 148 | 11 | 17 | 21% | 5 | 140 | 13 | 17 | 21% | 5 | 157 | 20 | 25 | 21% |
| Na+ | 217 | 2 450 | 512 | 574 | 15% | 164 | 2 069 | 447 | 507 | 15% | 160 | 3 419 | 560 | 644 | 15% |
| NH4+ | 2.5 | 12 332 | 1 348 | 1 776 | 15% | 31 | 11 804 | 1 089 | 1 516 | 15% | 48.4 | 11 104 | 981 | 1 502 | 15% |
| K+ | 37 | 6 173 | 244 | 333 | 15% | 27 | 1 314 | 179 | 243 | 15% | 40 | 9 301 | 346 | 507 | 15% |
| Ca2+ | 58 | 4 413 | 269 | 366 | 15% | 62 | 7 763 | 243 | 448 | 15% | 79 | 7 347 | 595 | 1 561 | 15% |
| Mg2+ | 2.5 | 357 | 39 | 48 | 15% | 2.5 | 1 063 | 35 | 57 | 15% | 2.5 | 1 838 | 74 | 179 | 14% |
| fluorides | 2.5 | 61 | 22.9 | 19.2 | 15% | 2.5 | 60 | 2.5 | 5.3 | 15% | 2.5 | 184 | 9.2 | 30.4 | 15% |
| chlorides | 23 | 3 360 | 256 | 437 | 15% | 26 | 4 315 | 208 | 422 | 15% | 44 | 14 329 | 759 | 1 505 | 15% |
| bromides | 2.5 | 37 | 10.3 | 11.5 | 15% | 2.5 | 18 | 2.5 | 2.8 | 16% | 2.5 | 42 | 7.7 | 10.2 | 15% |
| nitrates | 79 | 12 552 | 2 473 | 3 139 | 15% | 118 | 14 771 | 2 268 | 2 839 | 15% | 80 | 16 348 | 2 850 | 3 796 | 15% |
| sulphates | 19 | 18 044 | 2 072 | 2 781 | 15% | 12 | 16 355 | 1 626 | 2 340 | 15% | 378 | 17 889 | 2 883 | 3 634 | 15% |
| Na | 32 | 181 | 71 | 76 | 1% | 37 | 210 | 74 | 80 | 1% | 35 | 316 | 91 | 106 | 1% |
| Mg | 45 | 466 | 89 | 104 | 2% | 46 | 597 | 95 | 116 | 2% | 43 | 1 369 | 131 | 201 | 1% |
| Al | 0.5 | 1 835 | 0.5 | 38 | 2% | 0.5 | 1 802 | 0.5 | 49 | 2% | 0.5 | 2 118 | 0.5 | 52 | 2% |
| Si | 0.25 | 4 576 | 0.25 | 129 | 1% | 0.25 | 4 788 | 0.25 | 195 | 1% | 0.25 | 5 232 | 72.97 | 238 | 1% |
| S | 40 | 2 401 | 306 | 410 | 1% | 40 | 2 338 | 290 | 402 | 1% | 74 | 2 326 | 451 | 537 | 1% |
| Cl | 8.5 | 1 748 | 125 | 250 | 1% | 10.4 | 3 032 | 155 | 296 | 1% | 17.3 | 6 592 | 478 | 784 | 1% |
| K | 19 | 531 | 118 | 139 | 1% | 10 | 555 | 109 | 139 | 1% | 19 | 3 572 | 168 | 231 | 1% |
| Ca | 9.1 | 2 613 | 98 | 167 | 1% | 3.6 | 4 663 | 105 | 245 | 1% | 9.2 | 4 934 | 362 | 1 009 | 1% |
| Ti | 0.16 | 152 | 4.5 | 8.8 | 7% | 0.46 | 160 | 5.6 | 11.4 | 6% | 0.17 | 190 | 8.2 | 12.9 | 6% |
| V | 0.068 | 4 | 0.67 | 0.79 | 44% | 0.051 | 4.9 | 0.66 | 0.85 | 41% | 0.103 | 7.7 | 1.1 | 1.68 | 23% |
| Cr | 3.8 | 35 | 6.6 | 7.2 | 8% | 4.5 | 32 | 7.3 | 9.2 | 6% | 4.5 | 62 | 10.2 | 15 | 4% |
| Mn | 1.7 | 87 | 5 | 9 | 7% | 1.8 | 95 | 6.3 | 12 | 6% | 1.8 | 361 | 15.1 | 60 | 1% |
| Fe | 14.8 | 4 296 | 123 | 291 | 1% | 14.3 | 3 439 | 181 | 399 | 1% | 9.7 | 17 361 | 526 | 3 142 | 1% |
| Ni | 0.075 | 17 | 0.62 | 0.88 | 56% | 0.075 | 10 | 0.88 | 1.41 | 38% | 0.075 | 12 | 0.68 | 0.96 | 45% |
| Cu | 0.97 | 124 | 3.9 | 5.6 | 11% | 1.36 | 64 | 4.5 | 7.2 | 9% | 1.37 | 82 | 7.2 | 10.5 | 7% |
| Zn | 2.4 | 190 | 22 | 30 | 3% | 3.7 | 318 | 20 | 35 | 3% | 3.3 | 1 888 | 53 | 197 | 1% |
| As | 0.013 | 5 | 0.29 | 0.4 | 60% | 0.013 | 2.5 | 0.3 | 0.38 | 55% | 0.013 | 10.6 | 0.69 | 1.5 | 21% |
| Se | 0.01 | 4.2 | 0.76 | 0.89 | 56% | 0.01 | 3.4 | 0.77 | 0.92 | 52% | 0.03 | 4.1 | 0.92 | 1.05 | 50% |
| Cd | 0.14 | 17 | 5.8 | 6.4 | 203% | 0.14 | 24 | 5.8 | 6.2 | 210% | 0.14 | 156 | 5.9 | 8.4 | 167% |
| In | 1 | 148 | 10 | 12 | 192% | 1 | 49 | 10 | 12 | 167% | 1 | 51 | 10 | 12 | 192% |
| Sb | 0.86 | 271 | 20 | 29 | 10% | 0.98 | 447 | 20 | 34 | 9% | 1.61 | 506 | 49 | 95 | 5% |
| Ba | 0.24 | 32 | 2.3 | 3.4 | 17% | 0.14 | 30 | 3 | 4.6 | 13% | 0.36 | 32 | 3.5 | 5 | 14% |
| Pb | 0.72 | 50 | 8.1 | 10.8 | 13% | 0.9 | 65 | 6.8 | 9.8 | 12% | 1.28 | 379 | 19.2 | 46.2 | 4% |
| BaA | 0.35 | 65 | 4 | 8.2 | 26% | 0.32 | 91 | 3.2 | 7.7 | 25% | 0.37 | 224 | 12.5 | 24.3 | 25% |
| BaP | 0.19 | 41 | 3.4 | 6.1 | 25% | 0.17 | 62 | 2.7 | 5.8 | 24% | 0.2 | 182 | 8.8 | 18.9 | 25% |
| BbF | 0.32 | 34 | 3.3 | 5.4 | 26% | 0.31 | 45 | 2.7 | 5.3 | 25% | 0.3 | 147 | 9 | 16.4 | 25% |
| BghiPRL | 0.3 | 26 | 2.5 | 4.2 | 26% | 0.31 | 36 | 2 | 4.1 | 24% | 0.4 | 90 | 6.2 | 11.5 | 25% |
| BjF | 3 | 20 | 6.8 | 7.7 | 25% | 2.9 | 27 | 6.3 | 7.8 | 24% | 2.9 | 70 | 10.8 | 13.2 | 25% |
| BkF | 0.29 | 20 | 2 | 3.3 | 25% | 0.29 | 29 | 1.6 | 3.2 | 25% | 0.33 | 93 | 5.6 | 9.5 | 25% |
| DBahA | 0.59 | 2.3 | 0.82 | 0.96 | 25% | 0.59 | 2.4 | 1.04 | 1.12 | 25% | 0.6 | 12.6 | 1.93 | 2.51 | 25% |
| CRY | 0.34 | 60 | 5 | 8.9 | 25% | 0.3 | 79 | 3.7 | 8.1 | 25% | 0.42 | 189 | 13.1 | 22.2 | 25% |
| I123cdP | 0.58 | 22 | 2 | 3.4 | 25% | 0.6 | 31 | 2.1 | 3.6 | 25% | 0.59 | 49 | 5 | 7.7 | 25% |
| NOx | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | 4 | 124 | 31 | 37 | 15% |
| PM10 | 5.1 | 131 | 27 | 35 | 10% | 4.8 | 188 | 27 | 36 | 10% | 5.2 | 150 | 48 | 56 | 10% |

Fig. S4  PMF source apportionment of PM10 based on Fpeak Run.

Fig. S5 PMF source apportionment of benzo[a]pyrene based on Fpeak Run.

Fig. S6  Time series of absolute PM10 concentrations.

Fig. S7  Absolute contributions of concentrations of identified air pollution factors.

Fig. S8  Relative species concentration of identified air pollution factors.

Fig. S9  Polar plots, SIA factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S10  Polar plots, HEAT BB factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S11  Polar plots, HEAT CC factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S12  Polar plots, IND Ca factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S13  Polar plots, TRA factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S14  Polar plots, C-NO3-Na factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S15  Polar plots, Na-Cl factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S16  Polar plots, CRUSTAL factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S17  Polar plots, IND HM factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).

Fig. S18  Polar plots, IND PAH factor (concentration units in polarPlots are µg/m3, wind speed units are m/s).