Supplementary Materials for

1, 8- Cineole Extracted from Eucalyptus ecotypes’ Leaves: I. A Novel Microwave-Assisted Steam Distillation Method; Its Uploading into Natural Polymeric Encapsules for Pest Control

Sherif S. Hindi

Department of Agriculture, Faculty of Environmental Sciences, King Abdulaziz University (KAU), P.O. Box 80208, Jeddah 21589, Saudi Arabia; shindi@kau.edu.sa; Tel.: +96-656-676-0086

This PDF file includes:

Silica gel column

Essential oil

Saponification process

Methylation process

Hexane fraction

Elution with hexane

Gas Chromatography-Mass spectrometer (GC-MS)

Figure S1. Preparation of methyl esters of the essential oils for analyzing by GC-MS.

Table S1. Comparisons between extraction methods of essential oils.

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Hydro-distillation | Easy to use, selective, and flexible. | It's not feasible to get everything out; the process takes too long and uses too much energy. |
| Steam distillation | In contrast to hydro-distillation, less time is spent extracting, and fewer polar molecules are lost. | Longer extraction times result in non-appreciable and more expensive chemicals because of the extended processing times. |
| Organic solvent extraction | Simple, affordable, and moderately effective; suitable for small-scale operations. | Slow; requires a lot of solvent; prevents agitation from speeding up the process; uses organic solvents, which might have harmful or unexpected consequences on the end product. |
| Cold pressing | Easy and low-cost, it may be used for making citrus oils. | Failure to fully extract oil; impractical for low-oil samples. |
| Supercritical fluid extraction | Extraction without the use of harmful solvents takes less time and does not need their use. | Exorbitant prices for machinery, set-up, and servicing. |
| Microwave-assisted extraction | The extraction yield increased, and the process easily repeatable, manipulatable, need little solvent, required little energy, and energy efficient. | At the end, it will need to filter or centrifuge the mixture. |
| Ultrasound-assisted extraction | Quicker extraction, less solvent used, better mass transfer, deeper solvent penetration, and lower costs (in comparison to SCFE and MAE) all characterize this easy and cheap method. | Low performance, high energy efficiency, and poor scalability. |

Table S2. Statistical design of split-plot for studying the essential oil extracted from Eucalyptus hybrids grown at each of the campus of King Abdulaziz University (KAU), Hada Al-Sham, and Briman by using two extraction methods, namely microwave-assisted steam extraction (MASE), and electric-steam extraction (ESE) as repeated for three blocks.

| Block 1 |  | Block 2 |  | Block 3 |  |
| --- | --- | --- | --- | --- | --- |
| Main plot | Sub-plot (h) | Main plot | Sub-plot (h) | Main plot | Sub-plot (h) |
| KAU | MASE | Hada Al-Sham | MASE | Briman | ESE |
|  | ESE |  | ESE |  | MASE |
| Hada Al-Sham | ESE | Briman | ESE | KAU | MASE |
|  | MASE |  | MASE |  | ESE |
| Briman | ESE | KAU | ESE | Hada Al-Sham | MASE |
|  | MASE |  | MASE |  | ESE |

Table S3. Statistical design of split-split plot for the encapsulation investigation to study the difference between two polymeric encapsules, namely guar gum-based encapsules (GGBE) and alginate-based encapsules (ABE) which were uploaded with different concentrations of 1,8-cineol in a concentration of of 50, 100, 150 and 200 μL for a duration of 1, 1.5, and 2h as repeated for three blocks.

| Block 1 |  |  | Block 2 |  |  | Block 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Main plot | Sub-plot (h) | Sub-sub-plot (μL) | Main plot | Sub-plot (h) | Sub-sub-plot (μL) | Main plot | Sub-plot (h) | Sub-sub-plot (μL) |
| GGBE | 1h | 50 | ABE | 2h | 100 | GGBE | 1.5h | 200 |
|  |  | 100 |  |  | 200 |  |  | 100 |
|  |  | 150 |  |  | 150 |  |  | 50 |
|  |  | 200 |  |  | 100 |  |  | 1500 |
|  | 1.5h | 50 |  | 1h | 50 |  | 2h | 50 |
|  |  | 100 |  |  | 200 |  |  | 100 |
|  |  | 150 |  |  | 150 |  |  | 150 |
|  |  | 200 |  |  | 100 |  |  | 200 |
|  | 2h | 50 |  | 1.5h | 200 |  | 1h | 50 |
|  |  | 100 |  |  | 100 |  |  | 100 |
|  |  | 150 |  |  | 150 |  |  | 150 |
|  |  | 200 |  |  | 50 |  |  | 200 |
| ABE | 1h | 50 | GGBE | 2h | 100 | ABE | 1.5h | 150 |
|  |  | 100 |  |  | 200 |  |  | 100 |
|  |  | 150 |  |  | 50 |  |  | 50 |
|  |  | 200 |  |  | 200 |  |  | 200 |
|  | 1.5h | 50 |  | 1.5h | 50 |  | 1h | 50 |
|  |  | 100 |  |  | 200 |  |  | 150 |
|  |  | 150 |  |  | 150 |  |  | 200 |
|  |  | 200 |  |  | 100 |  |  | 100 |
|  | 2h | 50 |  | 1h | 50 |  | 2h | 150 |
|  |  | 100 |  |  | 150 |  |  | 100 |
|  |  | 150 |  |  | 100 |  |  | 50 |
|  |  | 200 |  |  | 200 |  |  | 200 |