Tables

Table 1: Comparison of different varieties of Vegetables found around different parts of the world

| Vegetable Waste | Pre-treatment Method | Temperature (°C) | Time (minutes) | pH | Enzyme | Comments | References |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Potato peels | Steam explosion | 160 | 10 | 4.5 | Cellulase | High starch content | (Sheikh et al., 2016; Sujeeta et al., 2018) |
| Carrot pomace | Dilute acid hydrolysis followed by enzymatic hydrolysis | 110 | 45 | 2 | Hemicellulase & Cellulase | Fermentation: Saccharomyces cerevisiae | (Khoshkho et al , 2022; Yu et al., 2013) |
| Tomato pomace | Ultrasound-assisted extraction | 80 | 20 | 5.5 | Pectinase & Cellulase | High moisture content | (Miriana et al., 2013) |
| Brinjal (Eggplant) peels | Steam explosion | 150 | 12 | 4 | Cellulase | Moderate lignin content | (Demiray et al., 2024) |
| Radish Leaves | Dilute acid hydrolysis | 100 | 60 | 2.5 | Hemicellulase & Cellulase | Requires optimization due to low sugar content | (Khan et al., 2015) |
| Onion peels | Steam explosion followed by enzymatic hydrolysis | 140 | 8 | 4 | Cellulase | Low cellulose content | (Genemo et al , 2021) |
| Pumpkin peels | Steam explosion | 150 | 12 | 4.5 | Cellulase | Suitable for co-fermentation with other wastes | (Chouaibi et al., 2020) |

Table 2: Comparison of different varieties of Lignocellulosic Biomass found around different parts of the world

| Feedstock | Pre-treatment Method | Temperature (°C) | Time (hours) | pH | Enzyme(s) | Comments | References |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sugarcane Bagasse | Steam Explosion | 160 - 190 | 1 - 10 | Nil | Cellulase | - Disrupts lignocellulosic structure for better enzyme accessibility. - Optimize steam pressure and residence time for efficient breakdown. | (Sabiha et al., 2018) |
| Corn Stover | Dilute Acid Pre-treatment | 90 - 130 | 30 - 60 | 1.0 - 2.0 | Cellulase, hemicellulase | - Similar to fruit peels, minimize sugar degradation and by-product formation. - May require additional enzymes to break down hemicellulose depending on process design. | (Riansa et al., 2011) |
| Switchgrass | Ammonia Fiber Expansion (AFEX) | 30 - 100 | 0.5 - 3 | Nil | Cellulase, xylanase | - Uses ammonia to swell the biomass and increase enzyme accessibility. - Requires proper ammonia recovery and wastewater treatment. | (Keshwani et al., 2007) |
| Wood Chips | Enzymatic Hydrolysis | 40 - 50 | 24 - 72 | 4.8 - 5.5 | Cellulase, xylanase | - Optimum temperature for cellulase and xylanase activity. - Enzyme cost can be a significant factor in process economics. | (Silva et al., 2011; Pereira et al., 2008; Vásquez et al., 2007) |

Table 3: A chart that compares the process of generating marine algae into bioethanol.

| Microalgal Biomass | Pre-treatment | Enzyme | Yield | Source |
| --- | --- | --- | --- | --- |
| Chlorococcum sp. | Enzymatic hydrolysis | Cellulase | 57% glucose (0.46 g/g bioethanol) | (Shokrkar et al., 2018) |
| Hindakia tetrachotoma ME03 | Enzymatic hydrolysis | β-glucosidase/cellulase, α-amylase | 92% saccharification | (Onay et a., 2019) |
| Chlorella sp. | Hydrothermal pre-treatment, Enzymatic hydrolysis | α-amylase, glucoamylase | 11 g/L bioethanol | (Ngamsirisomsakul et al., 2019) |
| Macroalgal biomass (fresh river water) | Enzymatic hydrolysis | Cellulase enzyme | 61% bioethanol | (Kumar et al., 2018) |
| Spirulina platensis | None (assumed) | α-amylase, amyloglucosidase | 80% polysaccharide conversion | (Rempel et al., 2019) |

Table 4: Comparison of different types of Marine Algae

| Algae Type | Pre-treatment Method | Disruption Type | Temperature (°C) | Time (minutes) | pH | Notes for Bioethanol Production | References |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Green Algae (e.g., Chlorella sp.) | Bead milling | Mechanical | 20-30 | 5-10 | Neutral | Disrupts cell wall for better sugar release during bioethanol production. | (Yadav et al., 2020) |
| Brown Algae (e.g., Saccharina sp.) | Dilute acid hydrolysis | Chemical | 90-100 | 30-60 | 1-2 | Break down complex carbohydrates to fermentable sugars for bioethanol production. | (Brown et al., 2024) |
| Red Algae (e.g., Gracilaria sp.) | Alkali pre-treatment | Chemical | 30-40 | 30-60 | 10-12 | Removes hemicellulose, improving access to carbohydrates for bioethanol production. | (Sehar et al., 2022) |
| Various Algae | Enzymatic hydrolysis (or other enzymatic methods) | Enzymatic | 40-50 | 60-120 | 4.5-5.0 | Breaks down cellulose and alginate for efficient conversion to fermentable sugars in bioethanol production. | (Jutakridsada et al., 2019) |

Table 5: Parameters considered for building the model

| Parameter | Expression | Description |
| --- | --- | --- |
| Microbial Growth Rate | dXdt=μmax∙SKs+S 1-[P][P]maxn-kd∙[X] | Rate of change of biomass concentration |
| Substrate Consumption Rate | dSdt=-1YXSdXdt-m.[X] | Rate of change of substrate concentration |
| Product Formation Rate | dPdt= -YPSdSdt | Rate of change of product concentration |

Table 6: Values of the other Parameters considered

| Parameter | Value |
| --- | --- |
| Ks | 1.7 kg/m³ |
| [P]max | 93 kg/m³ |
| n | 0.52 |
| kd | 12.108 exp-6860Th-1 |
| m | 0.03 h−1 |
| YXS | 0.08 |
| YPS | 0.45 |
| Initial concentration of Saccharomyces cerevisiae | 1.5 kg/m³ |
| Initial concentration of glucose | 220 kg/m³ |
| Fermentation temperature | 30 ℃ |

Table 7: Three diverse fruits and its various criteria

| Fruit | Content (grams/100g) | Method of Measurement | Other Notable Components | **Potential Sources |
| --- | --- | --- | --- | --- |
| Potato Peels (Raw) | 11.1 | HPLC | Dietary fiber, phenolic compounds, Vitamin C, potassium, iron, antioxidants | USDA FoodData Central, Healthline, WebMD |
| Sugarcane Bagasse (Dried) | 15.8 | HPLC | Lignocellulose, hemicellulose, lignin, trace minerals | USDA FoodData Central, Healthline, WebMD |
| Saccharina latissima (Sugar Kelp, Dried) | 11.4 | HPLC | Iodine, fucoidan, alginate, polyphenols, vitamins A, C, E, K, fiber | (Akter et al., 2024) |

**Data sourced from USDA FoodData Central, Healthline, and WebMD