Supplementary Materials

Table S1. Hardware configuration (UAV platform components and key specifications).

| Subsystem | Component | Key specification / justification |
| --- | --- | --- |
| Airframe | Tarot 650 Sport (TL65S01) | MTOM: ~3.65 kg; payload margin: ~1.2 kg; flight time: ~22–25 min. Retractable landing gear enables unobstructed 360° gimbal field of view. |
| Propulsion | Tarot 4114 320 KV motors + 15×5.5” props | Optimized for 6S efficiency (g/W) and long-endurance patrol; supports stable hover/slow cruise for surveillance and responsive control under gusts. |
| Power | 6S 22.2 V 10,000 mAh LiPo | Mixed-mode endurance ~22–30 min (mission-dependent). Battery current/voltage monitored (e.g., via Mauch sensor) for reliable power budgeting and safety. |
| Flight controller | Pixhawk Cube Orange+ (ADS-B Carrier) | IMU rate: 400 Hz (raw); EKF3 loop: 100 Hz. Interfaces: UART/CAN (MAVLink). Safety-critical ArduCopter 4.5.x (stable) with triple-redundant damped IMUs. |
| Edge compute (AI brain) | NVIDIA Jetson Orin Nano Developer Kit (8 GB) | JetPack 6.2 (L4T 36.4) / CUDA 11.8 / TensorRT 8.6 (FP16 & INT8). Ampere GPU with 1024 CUDA cores + 32 Tensor Cores (up to 40 INT8 TOPS) [27, 28]; power mode Max-P 15 W. Ethernet (RJ45) for RTSP ingest + UART for telemetry; clocks locked to prevent thermal-induced latency spikes. |
| Safety Island MCU | STM32H743ZI (Nucleo or custom board) | 480 MHz; 2 MB flash / 1 MB RAM. Interfaces: UART (from Jetson) + CAN (FD) (to Pixhawk). Runs deterministic tracking/control (TSMR + IMM-UKF) for fail-operational behavior under vision latency cliffs or link loss. |
| Camera + gimbal | SIYI A8 Mini (3-axis stabilized IP cam) | 4K (3840×2160) @ 30 FPS; bitrate ~15–20 Mbps; Ethernet (RTSP) + SBUS control. Primary vision input (onboard H.265 encoding to reduce bandwidth; data source for Table 6). |
| Onboard storage | SanDisk Extreme Pro microSDXC (128 GB) | Write speed ~90 MB/s (UHS-I U3). Used for high-bandwidth logging of video + telemetry to support replay, delay, and jitter experiments (Table 6). |
| Comms link (non-critical) | CubePilot Herelink HD Video TX/RX | 2.4 GHz ISM; dynamic bitrate (up to ~20Mbps); end-to-end latency ~110 ms. Used for monitoring (video + GCS telemetry); volatility motivates onboard dual-brain autonomy. |
| Ground station (GCS) | Laptop + QGroundControl + Herelink RC | Mission monitoring, situational awareness, and manual override via Herelink; link loss triggers onboard autonomous failsafe (Safety Island). |
| Protection | Marine hardening / waterproofing | Conformal coating on electronics (Edge AI computer, ESCs) and sealed enclosures/cable glands to mitigate salt spray, corrosion, and humidity in coastal operations. (Waterproofing ensures the UAV remains operable in rain/sea-spray, and all critical components are corrosion-protected.) |

Table S2. Edge platform comparison between NVIDIA Jetson Nano and NVIDIA Jetson Orin Nano for onboard maritime UAV perception deployment (key specifications affecting real-time Resi-YOLO throughput, power, and reliability).

| Attribute | Jetson Nano (4GB) | Jetson Orin Nano (8GB) |
| --- | --- | --- |
| GPU (CUDA / Tensor) | Maxwell, 128 CUDA cores; no Tensor cores [28] | Ampere, 1024 CUDA cores; 32 Tensor cores [27] |
| CPU | Quad-core ARM Cortex-A57 [28] | 6-core ARM Cortex-A78AE [27] |
| Memory | 4 GB LPDDR4 [28] | 8 GB LPDDR5 [27] |
| Memory bandwidth | 25.6 GB/s [28] | 68 GB/s typical; up to 102 GB/s (Super mode) [27] |
| AI compute (reference) | No TOPS rating (no Tensor cores); entry-level GPU compute [28] | Up to 40 TOPS (standard) / 67 TOPS (Super mode), model-dependent [27] |
| Power envelope | 5–10 W [28] | 7–15 W (configurable) [27] |
| OS / SDK | JetPack 4.x (legacy), CUDA/cuDNN/TensorRT (limited) [28] | JetPack 6.x, CUDA/cuDNN/TensorRT (recommended) [27] |
| Intended role in this work | Secondary baseline to show portability and performance trade-offs (Section 6.4) [28] | Primary target platform for real-time Resi-YOLO + tracking + MCU safety island integration [27] |
| Expected impact on Resi-YOLO | Reduced FPS and higher latency under 4K streams; still usable for constrained missions, depending on deployment constraints (resolution/FPS/latency budget) [28]. | Higher FPS and lower latency headroom for GSS-aware robustness tests and multi-stream workloads [27] |

Section S1. Tracking Robustness Protocol and Reproducibility Template

Section S1.1

Evaluation setting. We evaluate MOT robustness on SeaDronesSee-MOT sequences using TrackEval, reporting MOTA, IDF1, HOTA, and ID switches (IDSw). All runs use the same detector input size, confidence/IoU thresholds, and identical TrackEval settings to ensure fair comparison across video I/O conditions. For embedded deployment realism, inference is executed on a Jetson Orin Nano (TensorRT) edge platform [1], and the detector is exported to TensorRT when applicable [2,3,10].

Table S3. Experiment configuration template (corresponding to Table 6 in main text).

| Module | Parameter | Value (example) | Notes |
| --- | --- | --- | --- |
| Dataset | Sequences | SeaDronesSee-MOT (subset) | List sequence IDs and split (train/val/test). |
| Detector | Model | Resi-YOLO (ours) | Record weights hash and training epoch. |
| Detector | Input size | 640×640 | Kept constant across all Table 6 conditions. |
| Inference | Platform | Jetson Orin Nano (TensorRT) | Record JetPack/CUDA/TensorRT versions. |
| Tracker | Association | ByteTrack / Dual-brain IMM-UKF | Record thresholds and Kalman filter settings. |
| Replay impairment | Fixed delay Δt | +50 ms | Used in REPLAY-LAG50ms condition (Table 6). |
| Replay impairment | Timing jitter σ | 20 ms | Used in combined jitter+dropout condition. |
| Replay impairment | Frame dropout p | 5% | Used in combined jitter+dropout condition. |

Section S1.2

Impairment injection (replay). To emulate deployment degradations, we perform offline replay where detections (and/or frames) are passed through a controllable impairment layer before association: (i) fixed delay (Δt) and (ii) combined timing jitter (zero-mean Gaussian with std σ) plus frame dropout probability p. In Table 6, Resi-YOLO is evaluated under Δt = +50 ms (lag) and σ = 20 ms with p = 5% (jitter+drop). The dual-brain tracker’s TSMR mechanism is enabled during replay, allowing the MCU to retroactively correct past states when delayed measurements arrive.

Section S1.3

Command and parameter template (Ultralytics YOLO11 / Jetson Orin Nano).

Record all training, export, and inference parameters (model version, thresholds, and runtime accelerators) along with Table 6 results for full reproducibility.

- Jetson Orin Nano TensorRT export example: yolo export model=best.pt format=engine device=0 half=True

- Jetson Orin Nano inference example: yolo predict model=best.engine source=video.mp4 imgsz=640 conf=0.25 iou=0.7 save_txt=True

Replay + tracking example: python run_tracking.py --dets dets.txt --track_buffer 30 --ukf --tsmr --log_output

(Additional examples and logs, including raw detection CSV/JSON and tracking outputs corresponding to Table 6, will be provided in the project repository.)

Section S2. GSS Computation and Stratified Reporting (Script Outline)

Section S2.1

We compute GSS within a fixed upper-band ROI (top 65% of the image) to focus on sea-surface glare while excluding bottom-mounted platform artifacts.

Definition: For each frame, convert BGR to HSV and label glare pixels as glare = (V ≥ τ_v) ∧ (S ≤ τ_s) (with threshold values e.g. τ_v = 217, τ_s = 38). The Glare Severity Score (GSS) is then defined as the ratio of glare pixels within a predefined region of interest (e.g., the upper 65% of the image), yielding GSS ∈ [0, 1].

Section S2.2

Output: For each frame, store {frame_id, timestamp, gss, [optional glare mask stats]} to a JSONL/CSV log. Frames can then be grouped into low/medium/high-GSS strata to report AP/AR and tracking metrics as a function of radiometric clutter (glare level).

Section S2.3

Implementation (Python pseudocode):

# 1. def compute_gss(frame_bgr, v_thr=217, s_thr=38, roi_frac=0.65):

Glare Severity Score (GSS): fraction of glare pixels in the upper ROI.

roi_frac: fraction of image height kept from the top (default: upper 65%).

H = frame_bgr.shape[0]

roi = frame_bgr[0:int(roi_frac * H), :]

# 2. Convert to HSV color space

hsv = cv2.cvtColor(roi, cv2.COLOR_BGR2HSV)

# 3. Threshold glare pixels based on value and saturation

mask = (hsv[...,2] >= v_thr) & (hsv[...,1] <= s_thr)

# 4. Compute GSS as fraction of pixels in mask

gss = mask.sum() / mask.size

return gss  # (Optionally, return mask or mask stats as well)

https://developer.nvidia.com/blog/solving-entry-level-edge-ai-challenges-with-nvidia-jetson-orin-nano/

Section S3. Per-Stage Pipeline Latency Comparison Across Edge Platforms

This appendix reports a per-stage latency comparison between Jetson Nano (Maxwell) and Jetson Orin Nano (Ampere) to contextualize the hardware acceleration benefits observed in the main experiments. Measurements are obtained under identical streaming conditions with batch size = 1 and TensorRT FP16 deployment. The breakdown highlights improvements across decoding, preprocessing, inference, and post-processing stages, illustrating how architectural differences contribute to end-to-end performance gains. These results are provided for transparency and reproducibility and are not used to claim direct equivalence between platforms.

Table S4. Per-stage pipeline latency comparison between Jetson Nano (Maxwell) and Jetson Orin Nano (Ampere).

| Pipeline Stage | Jetson Nano (Maxwell) [ms] | Jetson Orin Nano (Ampere) [ms] | Speedup Factor |
| --- | --- | --- | --- |
| Capture & Encoding (Camera) | 55.0 | 40.0 | 1.38x |
| Network Transfer (RTSP) | 5.0 | 5.0 | - |
| Hardware Decode (NVDEC) | 35.0 | 15.0 | 2.33x |
| Pre-processing (VIC) | 22.0 | 8.0 | 2.75x |
| Primary Inference (Resi-YOLO) | 150.0 | 15.0 | 10.00x |
| Post-processing (NMS) | 12.0 | 4.0 | 3.00x |
| Messaging & Serialization | 6.0 | 2.0 | 3.00x |
| End-to-End Latency | ~285.0 | ~89.0 | 3.20x |

Section S4. Implementation details

The edge node runs containerized services for reproducibility. Video is decoded using a low-latency GStreamer pipeline from the RTSP stream. The model is exported to TensorRT (FP16 by default, with INT8 acceleration available) and the inference service outputs detection events after NMS. Optional geo-tagging uses autopilot telemetry (GPS/attitude) to associate detections with flight context. For reproducible evaluation, RTSP video, detections, and MAVLink telemetry are logged to high-speed onboard storage (Supplementary Table S1) and replayed to emulate delay/jitter conditions in a controlled manner (Table 6). The Jetson Orin Nano’s ample computing capacity also permits running multiple AI pipelines concurrently (e.g., a secondary classification model in parallel with detection) [4], which could be leveraged in future extensions.

As summarized in Supplementary Table S5, our Resi-YOLO design is portable to entry-level edge platforms (e.g., Jetson Orin Nano) and can still deliver usable tiny-vessel detection under constrained settings; however, the Jetson Orin Nano provides substantially more compute and memory bandwidth headroom, enabling higher-fidelity 4K streams, larger batch-free multi-stream operation, and more reliable robustness evaluation (GSS-aware replay/jitter/dropout) under realistic maritime communication uncertainty.

Supplementary Table S5. Edge deployment environment (Hardware/software configuration and runtime settings.)

| Item | Reporting fields / settings |
| --- | --- |
| Device | NVIDIA Jetson Orin Nano Developer Kit (8 GB) |
| Power mode | 15 W Max-P (Performance) mode; 6 CPU cores @ 1.5 GHz (A78AE); Ampere GPU 1024 cores @ up to 1.0 GHz (102 GB/s memory bandwidth in “Super” mode [6]). |
| Software | JetPack 6.2 (L4T 36.4.3) / Ubuntu 20.04 LTS / CUDA 11.8 / cuDNN 8.x / TensorRT 8.6; containerized deployment (Docker). |
| Inference runtime | NVIDIA TensorRT (via DeepStream SDK); precision: FP16 (INT8 calibrated capable); batch size: 1; GPU clocks locked; warm-up enabled. |
| Model input | 640×640 (default) / 960×960 (high-accuracy mode); NMS IoU = 0.45; confidence = 0.25. |
| Streaming decode | RTSP (H.265) from SIYI A8 mini camera; nvv4l2decoder with max-performance=1; low-latency buffering (latency=0, drop-on-latency=true). |