# Supplementary Information

for the article:

Significance of Time Value When Comparing Alternatives for Building Decarbonization

Additional Tables

Kate Chilton, Jay Arehart, Hal Hinkle

December 2024

Notes: The following information is intended to support the comprehension of the analysis contained within the article. References of the tables contained within this document can be found within the main document by searching for “Table S#.”

Table S1. Material inventory for the four framing systems.

| Framing System |  | Material No. | Material Name | Quantity | Unit |
| --- | --- | --- | --- | --- | --- |
| CMU | 1 | CMU block 8" x 8" x 16" | 1500 | EA |
|  | 2 | CMU bond beam block | 151 | EA |
|  | 3 | Rebar | 0.85 | TON |
|  | 4 | Grout fill | 768 | BAG |
|  | 5 | Cement | 920 | LBS |
|  | 6 | Type S hydrated lime | 40 | BAG |
|  | 7 | Sand | 4.25 | TON |
|  | 8 | Dimensional lumber | 1385 | LF |
|  | 9 | Anchor bolts | 77 | EA |
|  | 10 | Concrete 3500 PSI | 11 | CY |
|  | 11 | Connector screws | 19 | LBS |
|  | 12 | Framing nails | 6 | LBS |
| 2x6 | 1 | Dimensional lumber | 2836 | LF |
|  | 2 | Laminated veneer lumber | 16 | LF |
|  | 3 | Oriented strand board | 1358 | SF |
|  | 4 | Concrete 3500 PSI | 11 | CY |
|  | 5 | Rebar | 0.25 | TON |
|  | 6 | Anchor bolts | 52 | EA |
|  | 7 | Post bases and caps | 42 | EA |
|  | 8 | Hold-downs | 8 | EA |
|  | 9 | Framing nails | 37 | LBS |
| Bamboo- hybrid | 1 | Bamboo-hybrid panels | 3488 | SF |
|  | 2 | Dimensional lumber | 993 | LF |
|  | 3 | Laminated veneer lumber | 16 | LF |
|  | 4 | Concrete 3500 PSI | 11 | CY |
|  | 5 | Rebar | 0.25 | TON |
|  | 6 | Anchor bolts | 52 | EA |
|  | 7 | Framing nails | 69 | LBS |
| Euc-hybrid | 1 | Eucalyptus-hybrid panels | 3488 | SF |
|  | 2 | Dimensional lumber | 993 | LF |
|  | 3 | Laminated veneer lumber | 016 | LF |
|  | 4 | Concrete 3500 PSI | 11 | CY |
|  | 5 | Rebar | 0.25 | TON |
|  | 6 | Anchor bolts | 52 | EA |
|  | 7 | Framing nails | 69 | LBS |

Table S2. Summary of A1-A3 GHG emissions and biogenic carbon uptake per material.

| Material Name | Emissions Coefficient (kg CO2eq/unit) | Biogenic Carbon (kg CO2/unit) | Unit | Source |
| --- | --- | --- | --- | --- |
| Dimensional Lumber | 63 | 795 | m3 | CLF 2023 Baselines [27] |
| Cold-Formed Steel Framing | 2.44 | 0 | kg | CLF 2023 Baselines [27] |
| Laminated Veneer Lumber | 361 | 998 | m3 | CLF 2023 Baselines [27] |
| Oriented Strand Board | 243 | 1092 | m3 | CLF 2023 Baselines [27] |
| 3500 psi Concrete | 285 | 0 | m3 | CLF 2023 Baselines [27] |
| Rebar–- Fabricated | 854 | 0 | tonne | CLF 2023 Baselines [27] |
| Galvanized Steel | 1.71 | 0 | kg | EPD: Hot-Dip Galvanized Steel – American Galvanizers Association [38] |
| CMU Block | 240 | 0 | m3 | CLF 2023 Baselines [27] |
| Masonry Cement | 0.59 | 0 | tonne | CLF 2023 Baselines [27] |
| Masonry Grout | 0.17 | 0 | kg | Building Transparency EC3 [39] |
| Hydrated Lime | 1.10 | 0 | kg | EPD: Lime and Limestone Products – Boral [40] |
| Masonry Sand | 0.004 | 0 | kg | EPD: Durbin Sand & Gravel – Vulcan [41] |
| Nails | 0.94 | 0 | kg | Athena Impact Estimator for Buildings (IE4B) [42] |
| Bamboo-hybrid Panel | 2.38 | 3.075 | ft2 | EPD: Prime Wall System – BamCore [43] |
| Euc-hybrid Panel | 0.93 | 3.137 | ft2 | SimaPro |

Table S3. Summary scenario modeling parameters.

| Scenario No. | Scenario Name | Description | Growth Rate | Starting Year of Growth | Year of Max Adoption Rate | % Adoption of 2x6 |  | % Adoption of CMU |  | % Adoption of Bamboo-Hybrid |  | % Adoption of Euc-Hybrid |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Year 0 | Year 75 | Year 0 | Year 75 | Year 0 | Year 75 | Year 0 | Year 75 |
| 1 | Business as Usual | No change in the current makeup of structural systems used in new construction | N/A | n/a | N/A | 89% | 89% | 11% | 11% | 0% | 0% | 0% | 0% |
| 2 | Early-Slow | Adoption of fast-growing fiber-based systems starts immediately and grows at a slow pace | 0.3 | 2029 | 2050 | 89% | 35% | 11% | 4% | 0% | 30% | 0% | 31% |
| 3 | Late-Fast | Adoption fast-growing fiber-based systems is delayed but grows at a fast pace | 0.8 | 2062 | 2070 | 89% | 30% | 11% | 2% | 0% | 33% | 0% | 35% |
| 4 | Highly Optimistic | The level of adoption needed to achieve net-cooling by the end of the century occurs | 0.8 | 2025 | 2028 | 89% | 0% | 11% | 0% | 0% | 45% | 0% | 55% |