Table captions:

Table 1. Predictive modeling results for single spectral data

Table 2. Performance results of each model for fusion data

Table 1. Predictive modeling results for single spectral data

| Preprocessing methods | Data dimensionality reduction | Models | PCs |  | Calibration set |  |  |  | Prediction set |  |  | RPD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R2C |  | RMSEC |  | R2P |  | RMSEP |  |  |
| MSC | RF | SVR | 5 | 0.978 |  | 0.677 |  | 0.957 |  | 0.822 |  | 4.847 |
|  |  | PLSR | 9 | 0.868 |  | 1.658 |  | 0.799 |  | 1.766 |  | 2.261 |
|  | VCPA | SVR | 5 | 0.972 |  | 0.756 |  | 0.943 |  | 0.672 |  | 5.981 |
|  |  | PLSR | 7 | 0.925 |  | 1.233 |  | 0.950 |  | 0.887 |  | 4.533 |
|  | VCPA-IRIV | SVR | 7 | 0.994 |  | 0.350 |  | 0.973 |  | 0.693 |  | 6.119 |
|  |  | PLSR | 10 | 0.926 |  | 1.234 |  | 0.892 |  | 1.205 |  | 3.083 |
| SNV | RF | SVR | 4 | 0.901 |  | 1.419 |  | 0.914 |  | 1.219 |  | 3.456 |
|  |  | PLSR | 7 | 0.859 |  | 1.663 |  | 0.885 |  | 1.443 |  | 2.999 |
|  | VCPA | SVR | 7 | 0.965 |  | 0.846 |  | 0.964 |  | 0.807 |  | 5.273 |
|  |  | PLSR | 9 | 0.937 |  | 1.118 |  | 0.953 |  | 0.895 |  | 4.711 |
|  | VCPA-IRIV | SVR | 6 | 0.984 |  | 0.565 |  | 0.960 |  | 0.768 |  | 5.071 |
|  |  | PLSR | 9 | 0.901 |  | 1.401 |  | 0.909 |  | 1.200 |  | 3.376 |
| SMOOTH | RF | SVR | 4 | 0.960 |  | 0.895 |  | 0.935 |  | 1.079 |  | 3.940 |
|  |  | PLSR | 8 | 0.827 |  | 1.854 |  | 0.761 |  | 2.100 |  | 2.076 |
|  | VCPA | SVR | 5 | 0.926 |  | 1.213 |  | 0.909 |  | 1.337 |  | 3.345 |
|  |  | PLSR | 9 | 0.946 |  | 1.017 |  | 0.940 |  | 1.082 |  | 4.168 |
|  | VCPA-IRIV | SVR | 4 | 0.901 |  | 1.419 |  | 0.914 |  | 1.219 |  | 3.456 |
|  |  | PLSR | 10 | 0.921 |  | 1.267 |  | 0.907 |  | 1.258 |  | 3.335 |

MSC: multisource scattering correction; SNV: standard normalized variate RF: random frog VCPA: variable combination population analysis VCPA-IRIV: variable combination population analysis and iterative retained information variable SVR: support vector regression; PLSR: partial least squares regression; R2C: coefficient determination of the calibration set; R2P: coefficient determination of the prediction set; RPD: residual prediction deviation; RSMEC: correlation root mean square error; RSMEP: prediction root mean square error.

Table 2. Performance results of each model for fusion data

| Preprocessing methods | Data dimensionality reduction | Models | PCs |  | Calibration set |  |  |  | Prediction set |  |  | RPD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R2C |  | RMSEC |  | R2P |  | RMSEP |  |  |
| MSC | RF | SVR | 9 | 0.990 |  | 0.622 |  | 0.959 |  | 0.948 |  | 4.938 |
|  |  | PLSR | 9 | 0.888 |  | 1.442 |  | 0.889 |  | 1.547 |  | 3.051 |
|  | VCPA | SVR | 9 | 0.983 |  | 0.579 |  | 0.976 |  | 0.660 |  | 6.459 |
|  |  | PLSR | 10 | 0.951 |  | 0.987 |  | 0.950 |  | 0.941 |  | 4.534 |
|  | VCPA-IRIV | SVR | 10 | 0.995 |  | 0.312 |  | 0.986 |  | 0.558 |  | 8.155 |
|  |  | PLSR | 10 | 0.947 |  | 1.020 |  | 0.921 |  | 1.252 |  | 3.600 |
| SNV | RF | SVR | 9 | 0.989 |  | 0.480 |  | 0.962 |  | 0.851 |  | 5.008 |
|  |  | PLSR | 10 | 0.912 |  | 1.327 |  | 0.923 |  | 1.191 |  | 3.650 |
|  | VCPA | SVR | 9 | 0.995 |  | 0.323 |  | 0.970 |  | 0.761 |  | 5.854 |
|  |  | PLSR | 10 | 0.954 |  | 0.956 |  | 0.965 |  | 0.818 |  | 5.448 |
|  | VCPA-IRIV | SVR | 9 | 0.992 |  | 0.406 |  | 0.982 |  | 0.639 |  | 6.957 |
|  |  | PLSR | 10 | 0.903 |  | 1.373 |  | 0.925 |  | 1.233 |  | 3.704 |
| SMOOTH | RF | SVR | 8 | 0.981 |  | 0.623 |  | 0.950 |  | 0.942 |  | 4.530 |
|  |  | PLSR | 10 | 0.894 |  | 1.442 |  | 0.904 |  | 1.306 |  | 3.267 |
|  | VCPA | SVR | 8 | 0.976 |  | 0.710 |  | 0.942 |  | 0.972 |  | 4.051 |
|  |  | PLSR | 10 | 0.965 |  | 0.831 |  | 0.941 |  | 0.988 |  | 4.156 |
|  | VCPA-IRIV | SVR | 8 | 0.980 |  | 0.639 |  | 0.962 |  | 0.951 |  | 4.563 |
|  |  | PLSR | 10 | 0.940 |  | 1.095 |  | 0.925 |  | 1.132 |  | 3.693 |